QUICK INSTALLATION GUIDE

IDE/AT

Conner drives are * low level * formatted at the factory.

It is only necessary to run SETUP, FDISK  and DOS FORMAT.

Model     Hds  Cyls  Sec    Pcomp  L-Zone    Type    Table  LED

CP2034    2    823   38       0    823       *UT       3    n/a

CP2064    4    823   38       0    823       *UT       3    n/a

CP2084    8    548   38       0    548       *UT       3    n/a

CP2124    8    560   53       0    560       *UT       3    n/a

CP3184    6    832   33       0    832       *UT       1    A

CP3104    8    776   33       0    776       *UT       1    A

CP3204    16   683   38       0    683       *UT       2    B

CP30064   4    762   39       0    762       *UT       2    B

CP30084   8    526   39       0    526       *UT       2    B

CP30104   8    762   39       0    762       *UT       2    B

CP30084E  4    903   46       0    903       *UT       3    C

CP30174E  8    903   46       0    903       *UT       3    C

CP30204   16   683   38       0    683       *UT       3    C

CP3304    16   659   63       0    659       *UT       3    D

CP3364    16   702   63       0    702       *UT       3    D

CP3504    16   987   63       0    987       *UT       3    D

CP3544    16   1024  63       0    1024      *UT       3    B

Table 1 jumper settings:

Single drive = Jumper ACT and C/D

Master drive = Jumper ACT, C/D and DSP.

Slave drive = No jumpers installed.

Table 2 jumper settings:

Single drive = Jumper C/D

Master drive = Jumper C/D & DSP

Slave drive = No jumpers installed.

Table 3 jumper settings:

Single and Master drive = Jumper C/D

Slave drive = no jumper

ALL CONNER 20 MBYTE DRIVES USE DRIVE TYPE  2.

ALL CONNER 40 MBYTE DRIVES USE DRIVE TYPE 17.

* UT = Universal translate. Select a drive type that is close to,

but does not exceed the megabyte capacity of the drive. The drive

will translate to the megabyte capacity you have selected.

LED

A: J-4       B: J-1       C: J-5       D: J-3

Pin 1=+      Pin 3=+      Pin 3=+      Pin 3=+

Pin 2=-      Pin 4=-      Pin 4=-      Pin 4=-

QUICK INSTALLATION GUIDE

SCSI

Most SCSI host adapters  are compatible with Conner drives.

Software drivers and installation instructions are provided

with the host adapter. The drives are shipped with  SCSI ID

set to 7.  To select a different ID refer to the following:

Table A                       Table B

ID   E-1  E-2  E-3            ID   E2   E3   E4

0    out  out  out            0    out  out  out

1    in   out  out            1    in   out  out

2    out  in   out            2    out  in   out

3    in   in   out            3    in   in   out

4    out  out  in             4    out  out  in

5    in   out  in             5    in   out  in

6    out  in   in             6    out  in   in

7    in   in   in             7    in   in   in

Parity is always ENABLED on the CP3200,CP30060,CP30080,CP30100.

All other models, jumper E-4 to disable parity.

SCSI drive parameters:

Model          Hds       Cyl       Sec       Table     LED

CP2020         2         642       32        A         n/a

CP340          4         788       26        B         1

CP3020         2         622       33        A         1

CP3040         2        1026       40        A         1

CP3180         6         832       33        A         1

CP3100         8         776       33        A         1

CP30060        2        1524       39        A         2

CP30080        4        1053       39        A         2

CP30100        4        1522       39        A         2

CP30200        4        2119       49        A         2

CP3200         8        1366       38        A         2

CP3360         8        1806       49        A         2

CP3540         12       1806       49        A         2

LED 1              LED 2

J-4  Pin 1 = +      J-1 Pin 3 = +

Pin 2 = -          Pin 4 = -

CONTACT YOUR LOCAL DISTRIBUTOR FOR TECHNICAL ASSISTANCE

Revised : 28/9/91@20:14EST

(Some drives have a translation mode which gives them  a virtual

configuration different to their physical configuration. This is

to make the parameters lie within the BIOS limits -- 1024 cyls, etc)

Physical configuration.                 Translation Mode

Model    cap.     Cyl   Heads   Sec/trk     Cyl   Heads   Sec/trk

CP3024     21M      636     2       33        615     4       17

CP3044     43M     1047     2       40        977     5       17

CP30064    60M     1522     2       39        761     4       39

CP3184     84M      832     6       33        832     6       33

CP3104    104M      776     8       33        776     8       33

CP3114    112M      832     8       33        832     8       33

CP30104   120M     1522     4       39        756     8       39

CP3204F   212M     1366     8       38        683    16       38

CP3504    510M     1695    12       49        847    24       49

Note:  From my experience, its possible to use these drives in virtual

configurations different from both the physical config and the standard

'translation mode'.  e.g., I have used a CP3184 configured as

10 head, 965 cyl, 17 sec/cyl, and a CP3104 configured as 10 head, 1225 cyl,

17 sec/cyl.

Jumper settings.  ('J' => jumper installed, '-' => no jumper installed)

Jumper Description           1-Drive               2-Drives

Master        Slave

CP3024, CP3044, CP3184, CP3104, CP3114

ACT                          J                 -            -

DSP                          -                 J            -

C/D                          J                 J            -

HSP                          -                 -            J

CP30064, CP30104, CP3204F, CP3504

E1                           -                 J            -

E2                           J                 J            -

E3                           -                 -            -

This information comes from an "installation guide" from ACA Pacific Pty Ltd

(who are the Australian distributors for Conner hard drives).

===

File ends