A         L    .    .    .    .    .    .    .    .    .    .    .    R

|-2

|

|

|

|

|         [z][h]LANDMARK SOFTWARE[]

|            [z]#=1142 Pomegranate Court    Sunnyvale CA 94087    408-733-4035[][~LM]

|

[b]IMPORTANT:[]  SPEED and SETUP are neither public domain nor

shareware programs.  Rather, they are commercial programs.

The two programs are sold as a package for $29.95.  Companies

wishing to bundle either or both program should contact

Landmark Software for bundling prices.  Dealers should ask

about our DealerPak.  Corporate users should contact us about

multiple copy agreements and site licenses.

[b]What Systems Can Use SETUP?[]

SETUP is intended for AT's, AT-compatibles, and certain

AT-like 386 machines.  A message reminding you of this will

appear if you try to run SETUP on a PC, XT, PC Jr or any

member of IBM's Personal System 2 (aka PS/2) family.

[b]What Does SETUP Do?[]

On an AT class machine, setup parameters--specifying the

number and type of floppy & hard drives, the amount of

memory, etc.--are stored in battery-backed-up ram.  Each time

you change your system configuration or change your battery,

you have to go through the setup process.  With the setup

program that IBM provides, you have to reboot the system just

to determine the current setup values.  And you have to wade

through an annoying list of questions and answers.

Landmark's SETUP program eliminates all that:  You can

run SETUP at any time, without rebooting, to see or change

the setup parameters:  If you just want to see the current

settings, just run the program and you'll see all the current

settings shown on a single screen; you can then simply exit

from the program by pressing the [b]ESC[] key--there's no need to

reboot.  If you do want to make changes, you [b]just use the up

and down cursor keys to highlight the item you want to

change; the left and right cursor keys will then toggle

through the permissable values[]--while displaying the original

value to the left as a reminder of the original setting.

Except for the time & date, none of the other setup items

actually gets changed until you press the '[b]S[]' (STORE) key.

So there's plenty of opportunity for you to change your mind.

There's even an '[b]R[]' (REDISPLAY ORIGINAL SETTINGS) key that

let's you start over--all the tentative settings are changed

back to the way they were.

Within the program the [b]F1[] (HELP) key will display a

screen of help information.  And upon exit from the program a

message will appear that summarizes what's been done--and may

suggest a Print Screen or a reboot.

The command '[b]SETUP ?[]' will display a list of most of the

available command line options.

|

|                                     - 1 -

|

|

|-2

|

|

|

|

[b]Additional Information about SETUP[]

SETUP is intended to be self-explanatory.  There's no

installation procedure and no instructions are needed.  But

here are a few tidbits that may be of interest:

You'll want to keep SETUP on your hard disk to make it

convenient to check your setup parameters.  So you'll want to

put SETUP.COM in a directory that's on your path.  If you

also have other programs called SETUP, you may wish to rename

our SETUP.COM as ATSETUP.COM or LMSETUP.COM ('LM'=Landmark).

Because a battery failure will make your hard disk

inaccessible, we recommend that you keep a second copy of

SETUP.COM on a bootable floppy disk so you'll be able to get

your system back in operation quickly.

One of the nice features of SETUP is that it reads the

hard disk drive type table stored in ROM.  This lets you find

out what hard disk characteristics correspond to each drive

type number even if your machine's ROM definitions differ

from IBM's (as is the case with the Compaq 286) or has a

longer table of drive types (as is the case with the AST

Premium/286).  The abbreviation '[b]WPC[]' means 'write

precompensation cylinder'.

In rare instances, a machine's ROM may differ so much

from the IBM standard that SETUP is unable to locate the

drive type table and hence is unable to determine how many

drive types are supported.  In these cases the '[b]SETUP X[]'

command will permit you to specify the maximum drive type so

you'll be able to set the appropriate hard disk drive type.

Note, however, that since the program couldn't locate the

table, it can't read the specifications for each drive type

from the ROM:  You'll have to know the correct drive type

number from the documentation that came with your system.

Unlike most setup programs, Landmark's SETUP does not

require you to know what type of video card is in your

machine--we figure it out automatically based on the result

of the machine's Power On Self Test.  Just pay attention to

any special messages that SETUP may display.  For instance,

you may see a message telling you that the diagnostic status

byte is non-zero (which means that your current setup data,

in battery-backed-up ram, is invalid in some respect) and

that you should therefore use the '[b]S[]' (STORE) option even if

you aren't explicitly changing anything:  One of the effects

of the 'S' (STORE) option is that it makes the equipment byte

stored in battery-backed-up ram consistent with the equipment

byte defined by the Power On Self Test (POST).

Technically-inclined users may wish to try the '[b]SETUP D[]'

command which displays the actual contents of battery-backed-

up CMOS ram, including all the reserved fields.  This shows,

for instance, which of the two conventions for storing the

hard disk drive types is being used.  Also shows system &

model class.

|

|                                     - 2 -

|

|

|-2

|

|

|

|

|

[b]Changes in Version 1.13 (3-16-89)[]

Reduced the minimum legitimate number of cylinders from

306 to 256 to allow for the low cylinder counts that arise

due to sector translation with certain drives that in

actuality have more than 1024 cylinders; once again this

change was necessitated by an Advanced Logic Research (ALR)

system bios' hard disk table entry that specifies 266

cylinders, 63 sectors, 15 heads and a landing zone at

cylinder 1224.  The maximum legitimate number of cylinders

remains at 4096.

[b]Changes in Version 1.12 (11-2-88)[]

Expanded the maximum legitimate number of sectors per

track from 36 to 63 in order to accommodate Advanced Logic

Research (ALR) system bios' hard disk table that specifies 63

sectors per track for certain drive types intended for ESDI

drives using sector translation.  The purpose of sector

translation is to keep the apparent number of cylinders at or

below 1024.  See Version 1.11 changes for related discussion.

Whenever the number of sectors per track is different

than the standard 17, the actual number is now shown along

with the number of heads and cylinders.

[b]Changes in Version 1.11 (7-12-88)[]

SETUP reads the actual drive parameter table contained

in the BIOS of whatever machine it is being run on.  To

determine the number of drive types supported by a particular

BIOS, we look at each successive entry in the table until we

come to one that is neither all zeros nor a legitimate entry.

In determining the legitimacy of an entry, one of the tests

involves the number of sectors per track.  Previous versions

of SPEED required a number in the range of 17 thru 35.  We've

now expanded this range to 11 thru 36.

A table optimized for certain ESDI drives might

legitimately specify 36 sectors per track:  ESDI drives

usually have between 32 and 36 sectors per track, sometimes

depending on the controller as well as the drive.

The reason for accepting values below 17 is more

bizarre:  Many if not all recent DTK 286 BIOS's contain a

typo in the drive parameter table:  Type 30 erroneously

specifies 11 sectors per track instead of the correct figure

of 17 sectors per track.  This error by DTK caused earlier

versions of SETUP to think that there were only 29 drive

types in the DTK BIOS instead of the actual figure of 47

drive types.  We work around this DTK error by treating an 11

sector-per-track specification as legitimate for purposes of

determining the number of drive types.  When you select any

|

|

|                                     - 3 -

|

|

|-2

|

|

|

|

|

drive type that specifies fewer than 17 sectors per track,

however, you'll see '[b](INVALID SECTORS-PER-TRACK)[]' displayed

next to the drive type number.  This should be sufficient

warning that such entries should not be used.  This

modification enables SETUP to display the drive types beyond

the one that contains the erroneous sectors-per-track data.

[b]Changes in Version 1.10 (7-21-87)[]

You may now specify a [b]3.5" 1.44 meg drive[] as the type of

diskette drive.  In CMOS this is stored as a drive type of 4.

As before, the other permissable diskette drive types are

360K (type 1), 1.2 meg (type 2), 3.5" 720K (type 3), and 'not

installed' (type 0).  This is the only change in Version 1.10.

{              Please be aware that specifying the correct diskette

drive type setting is usually necessary, but NOT sufficient,

to make a 3.5" 1.44 meg drive operate properly.  And with

some 3rd party device drivers, you don't have to set the

diskette drive type at all--although this may depend on the

BIOS in your particular computer.

}

Unlike the case of 5.25" 1.2 meg (80-track) diskettes

vis-a-vis 5.25" 360K (40-track) diskettes, the 3.5" 1.44 meg

drives are supposed to read and write both 720K diskettes and

1.44 meg diskettes with no compatibility problems because

both have 80 cylinders--only the number of sectors per track

varies (9 sectors/track for the 720K vs. 18 sectors/track for

the 1.44 meg).  But not all 3.5" 1.44 meg drives are created

equal; some, like the Teac FD35HFN-30, "have no means of

identifying the density type of the installed diskette" while

others, such as the Sony MP-F73W series, "contain a

mechanical sensor to detect the existence of the extra hole

of a High Density (1.44 meg) diskette and can set itself to

the 1.44 meg mode automatically".

Different brands and models of drives may pose different

installation problems.  You may well have to make some

implausible changes to the jumper settings--such as removing

the FG jumper on the Teac FD35HFN-30 drive.  Depending on the

BIOS in your particular machine, you may be able to make the

drive function as a 1.44 meg drive without adding ANY device

driver to your CONFIG.SYS (for example, on an AST

Premium/286).  Even then, however, you may not be able to

handle 720K diskettes (the only kind produced by an IBM PS/2

Model 30, for instance).  Or you may find that you can't use

the drive at either capacity without a special 3rd party

device driver for 1.44 meg drives--instead of the DRIVER.SYS

driver that comes with IBM DOS 3.3.  Lastly, you'll need High

Density (HD) diskettes and a good deal of luck, particularly

without a 3rd party device driver.  As an example of how

widespread the problems are, we don't think DOS 3.3 by

itself, with or without using DRIVER.SYS, is sufficient to

|

|

|                                     - 4 -

|

|

|-2

|

|

|

|

|

make a 1.44 meg drive work in a genuine IBM AT (the original

6 MHz model).  And we certainly wouldn't place a bet on any

other model without trying it.

As of mid-1987, this is definitely the 'bleeding edge'

of technology.  For end users, we recommend buying your 1.44

meg drive as a kit that includes all necessary software and

comes with an unconditional money-back guarantee.  No vendor

has the resources to test every possible AT-class machine.

{              [b]Update Note:[]  Landmark can now provide 1.44 meg 3.5"

diskette drive kits for 386, 286 and 8088 class machines.

The least expensive upgrade is for a 286 or 386 machine that

currently has only one diskette drive; the kit for this

configuration includes a 1.44 meg drive with a grey bezel

which becomes drive B, a 5.25" mounting bracket already

installed, and non-copy-protected software to make it all

work:  a device driver for CONFIG.SYS and a program to format

1.44 meg and 720K 3.5" diskettes; except when formatting, you

don't have to specify the diskette capacity--the system

automatically detects the type of diskette that has been

inserted; $199 plus shipping with 10 day money-back guarantee

if it doesn't work in your machine.  If you'd like your

dealer to install it, we think a fair price would be $249

installed--have your dealer call us.  We also sell the

software separately for $75--works with Mitsubishi drives and

most others, but not Toshiba or Sony drives.  We can also

provide a card that lets you use up to four diskette drives

of any type in virtually any classic bus PC/XT, AT or 386

machine:  $149.

}

[b]Changes in Version 1.09 (6-29-87)[]

Our algorithm for determining the number of drive types

supported by a particular BIOS has been upgraded to allow for

the possibility of a sectors-per-track value other than 17

(for example, 25 or 26 for machines that include an RLL

controller as standard equipment, or perhaps 32 for machines

that come with an ESDI controller).  Our algorithm has long

allowed for the possibility of drive specifications that

differ from IBM's definitions even in the first 15 types (as,

for example, in the BIOS of the Compaq 286 machines).  And

for the possibility of 'all zero' specifications other than

for drive type 15 (for example, the AST Premium/286 has a

group of all zero specifications followed by additional non-

zero entries).  Our latest algorithm, fully implemented for

the first time in Version 1.09, will always determine a

maximum number of drive types provided it was able to find

the beginning of the table in the ROM BIOS.  And you can

override the resulting maximum number by using the '[b]SETUP X[]'

option which permits you to specify the maximum drive type.

The bottom line is that you may now be able to use just

'SETUP' on some machines that previously required 'SETUP X';

the major advantage in such cases is that 'SETUP' reads the

|

|

|                                     - 5 -

|

|

|-2

|

|

|

|

|

specifications for each drive type from the ROM BIOS

(allowing you to determine what each drive type actually

means on your machine) whereas 'SETUP X' always says

'specifications unknown' for each type.

When SETUP is run on a clearly inappropriate machine, a

message appears reminding you that SETUP is intended only for

AT's, AT compatibles, and certain AT-like 386 machines.  In

earlier versions of SETUP, this message was triggered for

PC's, XT's and PC Jr.'s.  With the introduction on 4-2-87 of

IBM's new line of PS/2 machines, we've now extended this so

that the message also appears for the so-called PS/2 Model 30

(an 8086 machine that differs drastically from other members

of the PS/2 family:  It can't take the same plug-in cards

because it doesn't have a Micro Channel bus, it can't run the

same video software because it has only a crippled version of

the VGA, it can't use the same diskettes because it has only

720K drives instead of 1.4 meg, and it can't run the OS/2

operating system because it has an 8086 instead of an 80286

or 80386).  And the message also appears for the PS/2 Model

50 (80286), Model 60 (80286) and Model 80 (80386); these

machines do not follow the original IBM AT layout of CMOS

data.  [b]If you'd like us to develop a version of SETUP for

IBM's PS/2 family, please let us know via a phone call.[]

We've also added a '[b]SETUP Z[]' option which bypasses checking

the type of machine; this is useful if you have a BIOS that

incorrectly identifies the type of machine; but please don't

use SETUP Z on an IBM PS/2:  You'll likely have to rerun

IBM's setup program to restore the original CMOS data.

{              The system class, model class and manufacturer (if

available) now appear on the first line of the diagnostics

screen that appears when the '[b]SETUP D[]' command is used.

}

[z][g]Landmark Software[]

[b]1142 Pomegranate Court[]

[b]Sunnyvale, CA 94087[]

[b]408-733-4035[]

|

|

|                                     - 6 -

|

|

|-