February 1993

LANsmart Installation

for

D-Link PCMCIA Ethernet Card

===========================

This installation procedure applies to the following D-Link products:

DE-650C,DE-650T

Files	     Contents

-----------  -------------------------------------------------

DE-650.DRV   Driver program for the above-mentioned adapters

DE-650.EXE   Driver program for the above-mentioned adapters

Note:

Before run LANSMART driver, run PCMCIA initial program on A:\OPEN_ADP

directory.

Please see as following:

*** PC installed Socket and Card services ***

Skip this step

OR

*** PC installed Socket service or Databook Cardtalk***

AE.EXE [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

OR

*** PC's PCMCIA controller is Intel 82365SL ***

PCIC.EXE [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

OR

*** PC is Toshiba T3300SL ***

ICCNT.EXE [/iop:P0] [/mem:P1] [/irq:P2]

*** For Phoenix PCM ***

DEVICE= PCMSS.EXE

DEVICE= PCMCCU.EXE

DEVICE= PCMDLINK.COM (irq:5, mem:d600, io base:320, only needed in

PCMCCU v1.3)

Please refer to \OPEN_ADP\README.DOC or \install.doc for detail information.

2. Parameters for OPEN-ADP utility:

PCIC.EXE

PCIC.EXE is used to enable Intel 82365SL PC Card Interface Controller

(PCIC) chipset.

ICCNT.EXE

ICCNET.EXE is used to enable ICCNT (I/O Card Controller) chipset in

TOSHIBA T3300SL Model Notebook.

AE.EXE

AE.EXE is used to enable PCMCIA Ethernet Card on which Socket Service has

been installed.

PCMDLINK.COM

PCMDLINK.COM is PCM client driver only for Phoenix PCMCCU v1.3 needed.

device=ae.exe [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

or

device=pcic.exe [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3] [/grp:P4]

or

device=iccnt.exe [/iop:P0] [/mem:P1] [/irq:P2]

device=pcmdlink.com (resources is fixed on irq:5, mem:d600, io base:320)

Parameter description:

/iop   :I/O base

/mem   :Attribute memory base

/irq   :IRQ value

/slt   :Slot type

/grp   :Group value

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable window memory base.

The default value is 0xd000.

P2 :The selectable irq value.

The default value is 0x5.

P3 :The selectable slot number.

The default number is 1.

P4 :The selectable grp value.

The default value is 0.

Group 0 means Index IO is 3E0h, Data IO is 3E1h, Base is 00h

Group 1 means Index IO is 3E0h, Data IO is 3E1h, Base is 80h

Group 2 means Index IO is 3E2h, Data IO is 3E3h, Base is 00h

Group 3 means Index IO is 3E2h, Data IO is 3E3h, Base is 80h

1. RUN LANsmart v.1.26 and v.1.30

==================================

Follow the procedure below to use the new LANsmart driver program on this

diskette with LANsmart v.1.26 and v.1.30.

1) Replace the LANsmart program LANMA05.DRV with the new driver program.

Example:

> COPY A:\LANSMART\DE-650.DRV  C:\LANSMART\LANSMA05.DRV

2) Now, you can use both the menu mode and the command mode to log onto

the network.  The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:5 [other parameters]

2. RUN LANsmart version 2.xx & v3.xx

=====================================

You may use both the menu and the command mode to logon to the network.

The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:DE-650 [other parameters]

The driver program DE-650.DRV should first be copied to the directory

that contains LOGON.EXE program. /NET:DE-650 specifies to LANsmart your

network adapter card.