User's Guide

QVCom:  QV-10 COMmunication software for LX

Version 0.52

JCG00070@niftyserve.or.jp

Masaki "Brahma" Tsumori

96/01/05

1 What is QVCom?

The goal of this program is to enable the data transfer between

HP100/200LX (or LX, as called throughout this document) and

digital camera QV-10, manufactured by Casio Computer Co.,Ltd.

(hereinafter called Casio).  This program is an EXM application,

same as the PIM of LX, and is used on the platform of System

Manager.

2  Content of the package

This package consists of the following files:

QVCOM.EXM    Program

QVCOM.DOC    This document

CABLE.DOC    Guide for electrical connection between LX and

QV-10.

CABLE.PCX    Attachment to CABLE.DOC

QVCOM1.ICN   Icon file for System Manager.  To be renamed as

QVCOM.ICN

QVCOM2.ICN   Icon file for System Manager.  To be renamed as

QVCOM.ICN

3  New features and changes from Ver.0.50

This version 0.52 has following new features, compared to the

previous Ver.0.50.

- Back scrolling in List screen.

- Block transfer from LX to QV-10.

- Saving to full color (24 bit color) bit map format.

4  Development history

When QV-10 from Casio first hit the market, its well-balanced

performance vs price and its new concept attracted many people.

I was among them.  All the LX users, who had a QV-10 in hand,

unanimously wanted, that LX and QV-10 should be connected

together.  Of course I shared the same desire.

I then wrote a letter to Casio asking for a possible access to

a technical documentation of QV-10, but never received a reply.

I imagine that the company naturally does not release any

information to an ordinary user who has no previous achievement

in the program development area.  I decided to make an

achievement in that area, worked on the analysis of the

communication protocol of QV10 by myself, and finally made this

program, QVCom.

* Note: After I released the previous version (Ver.0.50), I

wrote a letter to Casio again.  Its response was that

it will not disclose the technical specifications.

5  Installation

Following is the procedure of software installation. Please

refer to CABLE.DOC for electrical connection between LX and

QV-10.

1. Copy QVCOM.EXM to a preferred directory on your LX.

2. Press [&...] key to start More Applications.

3. Press [F2] key (ADD).

4. Enter the name of this application in Name: field (for

example:  QVCom) and the full path name of QVCOM.EXM in

Path: field (for example:  a:\exm\qvcom.exm).

5. Press [F10] key (OK). You will see an announcement panel

requesting you to assign a key for this application.

Press a key combination you would like to use for starting

QVCom (Example: ALT+F3)

6. Press [MENU] then [Q] to quit from More Applications.

This is the end of the installation.

If you are using MoreEXM (EXM launcher made by Mr.HIRO.), you

can register this program to MoreEXM.

If your are upgrading from Ver.0.50, all you need is to replace

QVCOM.EXM with the one in this package and reboot LX (CTRL+ALT+

DEL).

6  Usage

Current version of QVCom works on a user interface based on

function keys.

At start-up, QVCom has following function keys active.

[F8]  : List

Display thumbnail images of CAM-format files imported

in LX.  Data Directory defined in DefSetting menu is

searched for CAM-format files with a filename

QV10xxx.CAM, where xxx is any 3-digit number except

000.

12 images are shown on screen.  [ENTER] or [Down Arrow]

key will display next screen, [Up Arrow] key will

display previous screen and [ESC] key will quit from

List screen.

[F9]  : Connect

Connect LX to QV-10 and establish communication link.

[F10] : Quit

Quit from QVCom.

When the communication between LX and QV-10 is established by pressing

[F9] key, following function key will be activated.

[F5]  : Blk Get (Block Get)

Transfer plural CAM-format files from QV-10.  A

dialogue box will be displayed, prompting you to enter

the range of the files to be imported.  Specify by the

starting frame number and how many images you want to

get.  Imported CAM-format images are stored in Data

Directory as QV10xxx.CAM, where xxx is a sequencial 3-

digit number.

[F6]  : Get

Transfer one frame of image data from QV-10, in CAM

format or in temporary bitmap data.  Dialogue box

will appear for your entry of frame number.  The

following function keys are activated.

[F7]  : Show

Displays the image you specified by frame number

on color LCD screen of QV-10.

[F8]  : CAM

Import the image you specified by frame number,

in CAM format.  Imported CAM-format image is

stored in Data Directory as QV10xxx.CAM.

[F9]  : Cancel

Cancel and close the dialogue box.

[F10] : Get

Import the image you specified by frame number

as temporary bitmap data.

[F7]  : Send

Transfer CAM-format files from LX to QV-10. Dialogue box

will prompt you to select CAM-format file to send.

If you chose [All], it will send all the files with

.CAM extension (*.CAM) in the specified directory.

[F8]  : List

Display thumbnail images of CAM-format files imported

in LX.  This function is the same as in disconnected

condition.

[F9]  : Disconn (Disconnect)

Disconnect LX from QV-10.

[F10] : Quit

Quit from QVCom.  If you press this [F10] key under

connected status, QVCom will first close the established

connection and then quit, making it a little longer to

complete quitting.

When you have temporary bitmap data imported in your LX (by pressing

[F6] then [F10]), following function keys are activated.

[F3]  : Display

Display temporary bitmap data on LX. Dialogue box will

appear for you to specify display options:

- Size

4 sizes are available.  The first two sizes conform

to the aspect ratio of the display screen of LX.

- Halftone method

Selection between two methods for bitonal pseudo

grayscale presentation.

[F4]  : Save as

Save temporary bitmap data in LX, in monochrome PCX

format or in full-color BMP format.  A file with the

specified name will be created in Data Directory.

Pressing down [MENU] key will lead to the following menus.

[MENU]-[File]-[Exit]

Quit from QVCom.

[MENU]-[Setting]-[DefSetting]

Setting default configuration of QVCom.  The configured

values are stored in C:\_DAT\QVCOM.ENV.  This file is

automatically created when you first start QVCom.  You

can move this configuration file to A:\_DAT.

Dialogue box will be displayed to enter followings:

- BAUD

Communication line speed between LX and QV-10.  This

value is applied when you start QVCom.  The default

is 9600.  As high as 57600 is used successfully on

LX with double speed crystal.  Increasing the

communication speed will cause higher probability of

communication error.  This setting will take effect,

when you start QVCom next time.

- Data Dir

Define the directory to be used for storage of CAM

format files and other image files.

- Work Dir

Define the directory you use for holding temporary

bitmap data transferred from QV-10. The size of the

data is approximately 150 KBytes.  You need to

have enough free space to hold this amount.

[MENU]-[Setting]-[ComSetting]

Setting of communication line speed between LX and

QV-10.  The dialogue box will prompt you to select

the value.  This setting is valid in the current

session only and does not change the default value.

You can use this function only when the connection

is active between LX and QV-10.

[MENU]-[Quit]

Quit from QVCom.

7 Full color BMP file

QVCom creates uncompressed BMP files,  Image size 320x240 will

require approximately 230 KBytes, 480x240 image is 340 KBytes.

The raw data from QV-10 is a flattened image of 480x240 pixels.

QVCom compresses in horizontal direction to make 320x240 image,

which is slightly blurred.

The raw data, without color compensation, has color tones

different from the one obtained by genuine Link Software from

Casio.  Activating color compensation will give images with

colors close to the genuine software.  Raw image has 256 tones

in each of RGB components.  This will be reduced to

approximately 180 tones after compensation.

The richest information is obtained with the size set at

480x240 and without color compensation.  Please use this mode

if you further modify the image by a program such as PhotoShop.

8 Important Notice

When you use QVCom, you should observe followings.

- Task swap (starting another EXM application) is disabled when

LX and QV-10 are connected.  You can swap when they are

disconnected.

- When odometer halts in the middle of data transfer, QVCom is

conducting recovery process.  Please be patient and wait

until communication error dialogue is displayed.

- If you are using disk compression program such as Stacker,

actual free space can be different from what you see. In

this case you possibly encounter errors even when enough

space is seemingly available according to the display.

- If data transfer is terminated by communication error, the

incomplete file is automatically deleted. This should make

transfer errors absent from the CAM files already imported

into LX.  Unfortunately, however, this is not 100% assured,

since check-sum will not detect every errors.  QVCom has no

capability to decompress and display CAM files.  If you

deal with very important data, you need to transfer it

back to QV-10 to test its integrity, prior to its deletion

from QV-10.

9 Known problems

A case is confirmed where no recovery is possible from

communication error during data transfer.  It happens when the

odometer is at 0%.  Reboot (CTRL+ALT+DEL) is required.

When you receive CAM-format files from QV-10 while listing of

images is on the screen, then you have a possibility of

mismatch between the images on the screen and actual data

stored in Data Directory.  This is intentionally left as it is,

since eliminating the mismatch would cause too frequent

refreshing of the screen - every time other action is completed.

10 Development plan

Following are what will be possibly considered in future.

- Enhancement of Listing screen

List screen currently shows only a preview thumbnail of each

CAM files. Adding the capability of showing and editing the

comment field of CAM files are considered.

- Decompression of CAM-format files

Compressed image data of CAM file currently cannot be expanded.

A possibility exists, that the decompression will be

implemented, although it is not practical to do so on LX (it

will take 2 to 3 minutes to make one image decoded).

- Operation of QV-10

There is an idea, that enabling the operation of QV-10 from a

remote LX will be interesting for the purpose of business

presentations.

11  Communication protocol

QVCom is based on a communication protocol I deducted from the

analysis of the communication between PC and QV-10 and based on

the commands revealed by various tests (and by accident too).

I will not disclose this protocol, since there are no assurance

that it is correct.

I must add here, that if Casio discloses the communication

protocol, the quality and functions of QVCOM will surely be

improved.

12  Development tools

This program was developed with Mr. Nakamoto's NKIT Ver.0.61

and Borland TC++ Ver.1.0 2nd Edition for PC98.  (The executable

in this package is compiled by this complier).

The source code will not be released at this time.

13  About qvplay

QV-10 Link software (qvplay) for UNIX was developed by Mr

Kenichi Hayashi and Junichiro "Itojun" Ito and already made

available.  Qvplay uses the communication protocol of its own

analysis, and works on various UNIX platforms from workstations

to PC.

As of today (96/01/05) the latest version is on the following

URL:

ftp://ftp.csl.sony.co.jp/pub/QV10/unix/QVplay-0.07.tar.gz

[  Translator's note: as of 1996-04-03 the newest version is:   ]

[  ftp://ftp.csl.sony.co.jp/pub/QV10/unix/qvplay-0.09.tar.gz    ]

[                                             (21-mar-96, 61K)  ]

You can get it also from Web site of Mr Hayashi:

http://www.asahi-net.or.jp/~XG2K-HYS/

14. Condition of usage of this software

Usage of this program is permitted to those who observe the

following conditions.

- This program is photoware.

- I (Masaki Tsumori, Brahma) own the copyright of this program.

- Users of this program have no obligations to pay for it, except

for the fee to the author, paid if the user likes it.

- Giving this program to other people is permitted under the

condition that those who receive it  understand that the

program is beta test version.  Public distribution to

indefinite numbers of people is not permitted since this is a

beta test version.

- You should not put this program to BBS nor to FTP site nor

other similar places, since this is a beta test version.

- Mention of this software in periodicals and literatures can be

permitted if prior notification is made.  In some cases it

will not be permitted.   Including this program in the

appendix of books or periodicals is principally not permitted.

In some cases it will be permitted.

- The author has absolutely no responsibility for any kind of

damages which may be caused by this program.

15  Photoware

This program is a photoware.  If you like this program, send

a photograph or photographs to the author.  Of course they

should be taken by QV-10.

Humans, animals, objects, anything can do.  Show me by them

that you are enjoying QV-10 and that QVCom is helping you to do

so.  This gives me a motivation for further program development.

The photographs sent to me will be kept completely confidential

and will never be disclosed.

The photographs should be sent in CAM format, by binary mail to

NIFTY:JCG00070

through X.400 gateway between Compuserve and NIFTY-Serve. (For

details, please refer to HELP in Compuserve MAIL)

If you have difficulties sending binary mail, please use ISH or

UUENCODE or other ASCII encoding software to convert the images

to ASCII format and send them to the same address or

JCG00070@niftyserve.or.jp

through Internet.

I will not reject (rather do welcome <G>) donations, which are

used for purchasing of books, development tools or the future

generations of QV-10.

16  literature.

Following literature are used for reference while I developed this

program.

- Agui et al:  C Gengo niyoru Gazo Shori, published by Shokodo,

Tokyo, 1990, ISBN 4-7856-3076-0

- Ono, Suzuki:  Wakariyasui JPEG/MPEG2 no Jitsugenho, published

by Ohm Sha, Tokyo, July 1995, ISBN 4-274-07819-1

- Yagi et al:  C Gengo de Manabu Jissen Digital Eizo Shori,

published by Ohm Sha, Tokyo, 1995, ISBN 4-274-94056-X

- Contrail:  DIB(BMP) Format no Kozo Setsumeisho, downloaded from

NIFTY-Serve, Forum FGALAV, Library No.7, File No.590.

[Translator's note: current location is: FGALAV. Library 2,

File #22]

17  Acknowledgement

I would like to express my deep thanks to Mr.Nakamoto, whose

NKIT I used for the development of this program.

Alpha test of this program was conducted under the kind corporation

of Mr.NORI, Mr.Kenichi Hayashi, Mr.HEI, Mr.LEF, Mr.Chogokin, Mr.

TAD31, Mr.KUI and Mr.kikuyan.  My deep thanks are also for them.

I deeply thanks to Mr Kazushi Kimura, who gave me an idea for the

implementation of this program.

Lastly I would like to extend my thanks to the members of Nonbe

Leagues and Hokkaido Banquet Unions who sometimes supported and

sometimes disturbed the development of this program which

continued for such long time.

18  Appendix

I was very pleased by the reaction from a lot of people when I

released the beta test version of QVCom.  However, the denial

of disclosure of information from Casio is so discouraging that

now I have no plan to work on new versions of this program.

This program is supported on Section 7, Forum FHPPC of NIFTY-

Serve(*).

[  Translated from Japanese by:       ]

[  Toshiki Sasabe (Janbe) 73560,524   ]

* Translator's note:

Feed back is also welcome in Section 11,  HPHAND forum,  on

Compuserve.  I will do my best to transfer the comments to

the author.