SETCOM1 V1.0

(c)1999 Stefan Peichl, Heidelberg, Germany

http://home.t-online.de/home/stefan.peichl/

eMail: stefan.peichl@t-online.de

SETCOM1 is a mixture of the built in programs SERCTL and MODE.

It allows you to confire the serial port COM1 for higher baud

rates than it is possible with the MODE command, which is

restricted to baud rates up to 9600.

If you call SETCOM1 without parameters, it displays the current

setting of the COM1 port and the help screen of SETCOM1.

If you want to configure COM1, you may use the following

command line switches:

1=   300 baud

2=  1200 baud

3=  2400 baud

4=  4800 baud

5=  9600 baud

6= 19200 baud

7= 38400 baud

8= 57600 baud

9=115200 baud

e= e,7,1 ;even parity, 7 data bits, one stop bit

n= n,8,1 ;no   parity, 8 data bits, one stop bit

i= select ir   channel and power it up

w= select wire channel and power it up

o= power off ir and wire channel

The command line switches are not case sensitive and may be

used in any order with or without separators. For example

SETCOM1 9nw

SETCOM1 W/9/N

SETCOM1 w /N,9

are all identical and set COM1 to:

COM1: 115200,n,8,1,wire,on