Enabler for the QUATECH SPP-100

SPP100.EXE Version 0.20

Copyright (C) 1996 Hiroyuki Sekiya

[ What is SPP100.EXE? ]

SPP100.EXE is an enabler to be used with the SPP-100, a

parallel port PC card from QUATECH, on the HP 100/200 LX.

The SPP-100 card is supplied with SPP100CL.COM, an enabler

for the HP 100/200LX.  SPP100.EXE has following advantages

over SPP100CL.COM.

o Can be loaded both as a TSR program and as a device driver.

You can now use other drivers, which requires a parallel

port being already enabled within CONFIG.SYS.

o Enables SPP-100 ONLY

On the other hand, SPP100CL.COM tried to enable whatever

on the card slot, even a ATA card, as a parallel port.

o Can enable in EPP mode (from Version 0.20)

It can enable the card in EPP (Enhanced Parallel Port)

mode. You can enjoy the high speed data transfer with

SCSI adapters and Ethernet adapters, which support EPP

mode. (EPP BIOS is not supported, so some equipment may

not work in EPP mode)

[ Hardware Requirement ]

PC:  HP100LX or HP200LX

PC Card: QUATECH SPP-100 (no other card is supported)

[ How to use it ]

Follow the procedures below:

0. Copy SPP100.EXE to C: drive (any directory)

1. Terminate all applications and turn the power off.

2. Insert SPP-100 to the card slot.

3. Press [CTRL]+[SHIFT]+[ON] (hard reset)

4. Press [N] to "initialize C drive?" display.

5. Start the system from C or D drive.

6. Load SPP100.EXE.

SPP100.EXE functions both as a device driver and a TSR

program. You can place it in your CONFIG.SYS or AUTOEXEC.BAT.

When you use drivers such as ASPI manager for a Parallel

SCSI adapter, requiring an accessible parallel port while

being loaded within CONFIG.SYS, you should load SPP100.EXE

earlier than such a driver.

If you load SPP100.EXE as a TSR program, you can unload it

with '-r' option.  If loaded as a device driver, it cannot

be unloaded,

Following options to SPP100.EXE control the mode of

parallel port.  You can change the mode of driver already

loaded.

-u      Uni-direction (default)

-b      Bi-direction

-e      EPP (Enhanced Parallel Port)

Default setting is the unidirectional mode.  Switching to

the bidirectional or to the EPP mode will enable a high

speed transfer in some devices such as SCSI adapters.  Some

other devices, such as ZIP drive, can be recognized only in

unidirectional or bidirectional mode (no EPP-mode).  You

should first test by changing this option.

[ Application of parallel port of 100/200LX ]

Following devices, other than printers, have reported to

work with this parallel port card.

[zip drive]

Parallel Zip Drive (IOmega)           Bi-direction

[SCSI Host Adapter]

Mini SCSI Plus (Adaptec/Trantor)      Bi-direction

BRIDGE 777EP                          EPP

BRIDGE 888                            EPP

[Ethernet Adapter]

Pocket-Sized Ethernet Adapter         Bi-direction

Pocket Ethernet Adapter II (Xircom)   Bi-direction

[2.5inch Hard Disk Adapter]

MD2                                   ?

[ Message from the author ]

This version does not support EPP (Enhanced Parallel Port) BIOS,

I found a number of devices does not work in EPP mode. I would

like to work on its support in future.  If you know where to

find technical information in this area, please let me know.

[ Copyright and Distribution policy ]

The copyright of this program is owned by its author Hiroyuki

Sekiya.  You are permitted to use it freely, if it is for

your private use.

Distribution of this program modified from the original is

prohibited, regardless of the degree of the change.

Distributing of this software, whether in part or in a

whole without the prior permission of the author is

strictly prohibited.  Distribution includes, and is not

limited to, placing the software in BBS or on ftp site,

bundling it in a storage media attached to a publication,

bundling it in distribution media of other software, and

similar matters.

[ History ]

0.10      First release.

0.20      From this version, EPP registers can be enabled.

[ Where to contact ]

Report on usage and bugs are accepted in Section 7 of the FHPPC

forum on NIFTY-Serve.  If you have difficulties in posting a

message to NIFTY-Serve, you can send e-mail to the following

address.  Your messages written in languages other than

Japanese may not always be replied.  Inquiry about distribution

or similar communication should be also directed to this

address.

JBD00432@niftyserve.or.jp

Hiroyuki Sekiya

[  Translated from Japanese by:  ]

[  Toshiki Sasabe (Janbe)        ]

[  73560.524@compuserve.com      ]