Enhanced VGA DRIVER INSTALLATION

FOR MICROSOFT WINDOWS 3.0 and 3.0a

The instructions which follow are applicable to users of Microsoft

Windows and Enhanced VGA card (based on Tseng ET4000 chips).

Note that, to use 800x600 , 1024x768 and 1280x1024 modes, an ANALOG

color display capable of these high resolutions is required.

The "Enhanced VGA - Windows 3.0 Driver" diskette contains 256-color

and high-resolution drivers, as well as a file called SETUP.INF

that replaces the file of the same name on the Microsoft Windows

3.0 or 3.0a Setup "DISK 1" diskette. Follow the instructions listed

below carefully to install the drivers.

Instructions:

1. Make a DISKCOPY of the Setup "Disk 1" diskette.

Example (for systems with a single floppy and hard drive):

C:>DISKCOPY A: A:

The system will use drive A: for both the original and the

copy as the source and target diskettes are inserted

alternately for the copy.

2. After the copy is made, return the original to its jacket and

store safely.

3. Copy the SETUP file from the "Enhanced VGA - Windows 3.0 Driver"

diskette to the newly-created copy of the Setup "Disk 1"

diskette. Example (for systems with a single floppy and hard

drive):

For Windows 3.0:

C:>COPY A:SETUP.W30 B:SETUP.INF

For Windows 3.0a:

C:>COPY A:SETUP.W3A B:SETUP.INF

The system will use drive A: as drive B: also. After the copy

is finished, type A: [ENTER].

4. Type SETUP [ENTER] to begin installation of Windows.

5. Review the configuration list when it is shown. If you wish to

change Display type, move the cursor (highlighted line) to the

"Display:" line and press [ENTER] to select from the available

list.

6. The program will ask you to insert the "Enhanced VGA - Windows

3.0 Driver" diskette in order to install the drivers as required.

After  this is done,  the installation program  will continue

normally.

After you complete the above steps you can begin using Windows.

Note: If you wish to change the display type after completing the

installation, click on the Windows Setup icon found in the Main

desktop window.