DSZ - a ZMODEM, True YMODEM(TM), XMODEM File Transfer Program

Chuck Forsberg

Omen Technology Inc

Omen Technology Incorporated

The High Reliability Software

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

Modem: 503-621-3746 Speed 1200,2400,19200(Telebit PEP)

Compuserve:70007,2304  GEnie:CAF

UUCP: ...!tektronix!reed!omen!caf

Chapter 0             Rev 10-19-88  Typeset 10-19-88                     1

Chapter 0                  DSZ Quick Reference                           2

1.  IN CASE OF DIFFICULTY

If you have difficulty getting DSZ to work, start with the following

command to DSZ directly from the DOS prompt, without running any TSR or

other COMM program:

dsz port 1 speed 2400 d t

(Change the port number and speed as necessary.) This connects you to the

modem.  Dial a bulletin board by typing AT commands to the modem

("ATDP621-3746").  Log in as usual.  Then, request a ZMODEM download of a

file you don't already have.  The file will download automatically, you

don't have to do anything.  This procedure verifies the correct operation

of DSZ.

The "sb" and "sz" commands exploit DOS's tree structured file system.  An

"sz" command with no file specified sends all the files in the current

directory.  A directory or disk specifier expands to all files in that

directory.

1.1  Incorrect Batch Files

Incorrect batch files are the number one source of problems using DSZ for

downloads.  DSZ's command line echo was added to assist in correcting

defective batch files.  Try running DSZ by itself, without any batch files

or TSR programs, calling it directly from DOS as described above.

1.2  No Carrier Detect

If you see a No Carrier Detect message, you really need to give the d

command:

dsz port 1 speed 2400 d t

Better yet, correct your modem cable to allow proper operation of the

Carrier Detect signal (CD)!

Some BBS and/or "door" programs hang when the program (such as DSZ) they

call returns with a non 0 exit status.  Please refer to the "EXIT STATUS"

chapter of the document for more information.

1.3  PKARC 3.6

It has been reported that PK36 grabs some interrupts and doesn't restore

them on exit in an attempt to prevent hacks of the program.  Since

eliminating that newer version and cleaning up memory, highspeed xfers

even with 8250 UARTS work properly again.

The problem with PK 3.6 appears to be most severe if a BBS operator has

doorware, etc that accesses the PK programs from a shell of some sort.

This info may be helpful to users who are having otherwise inexplicable

file xfer problams.

Chapter 1             Rev 10-19-88  Typeset 10-19-88                     2

Chapter 1                  DSZ Quick Reference                           3

1.4  COMMAND NOT FOUND

If you see this message, make sure DSZ is in the DOS PATH.  Sometimes this

error is caused by having too many environment variable strings.

1.5  DOS 3.x

DOS 3.x systems may need to have a "STACKS = 8,256" line in CONFIG.SYS to

prevent antisocial behavior. This is required on the NEC Multispeed, or if

certain TSR's are used.

1.6  Lockups

DSZ is fastidious about which interrupt vectors it takes over.  Unlike

some COMM programs, DSZ grabs only the correct interrupt vector for its

interrupt service routine.  This allows multi line operation under

DESQview and other multitasking systems.

Incorrectly configured serial ports that work with programs that pig out

both IRQ3 and IRQ4, will lock up with DSZ.  If DSZ locks up the machine as

soon as it tries to send or receive data, this is usually the fault.  Make

sure the comm port is strapped correctly; COM1 uses IRQ4, COM2 uses IRQ3.

DSZ will lock up if the comm port is not properly set up.

In addition, DSZ will not work properly if two devices are connected to

the same IRQ line, even when some other programs (that don't use

interrupts as intensively) do work normally on the same machine.

1.7  IBM PS/2 Model 50Z

Early samples of this machine have a design error on the motherboard that

causes comms programs to lock up the machine.  Replacing the motherboard

with a correctly functioning unit from current production corrects the

problem.

1.8  Brain Damaged UARTS

Omen Technology has received reports of problems with defective 8250 type

UART integrated circuits in Leading Edge modem boards, serial port

interfaces, and computers.  The defective chip logic affects high

performance software.  Replacing the buggy chip with a newer chip (16450

or NS16550AN) corrects the problem.  The NS16550AN is a plug in

replacement for 8250 and 16450 serial chips.  386 and fast 286 machines

should use the NS16550AN for best results.) Arrow Electronics 1-800-932-

7769 sells National NS16550AN chips mail order (credit card) for about $22

each in small quantities.

Chapter 1             Rev 10-19-88  Typeset 10-19-88                     3

Chapter 1                  DSZ Quick Reference                           4

1.8.1  Everex Internal Modems

Numerous reports have been received concerning difficulties with Everex

modem cards.  Some of these cards contain buggy 8250 chips which are not

up to the demands of full duplex streaming protocols such as ZMODEM.

Replacing the buggy 8250 chip with a 16450 (or better yet NS16550AN) chip

corrects these problems.

1.9  Weird Iron

A few machines, some with V20 CPUs, don't like DSZ.COM.  We have not been

able to repeat this; TeleGodzilla (using a V20) runs DSZ.COM without a

hitch.  Switching to DSZ.EXE has cleared up the problems on other

machines.

1.10  Will the real YMODEM please stand up?

Some XMODEM programs claiming to support YMODEM do not send or accept the

YMODEM pathname block.  1024 byte XMODEM is XMODEM-1k, not YMODEM as

defined by Ward Christensen in 1985 when he coined the word.  1987 and

later versions of YMODEM.DOC remind the reader of the official YMODEM

definition, which has not changed since YMODEM's coinage in 1985.

1.11  286 and 386 Extended Memory

Virtual memory disk caches and ramdisks using 80286 or 80386 extended

memory require a kluge hardware reset of the CPU chip to return to the

normal "real mode" after accessing extended memory.  This interference to

the interrupt response time causes lost characters on comm ports, and may

be minimized by specifying shorter memory transfers.

Switching to a National NS16550AN UART chip helps tremendously, allowing

transfers at 115200 bps instead of 9600 bps with a standard 8250.  The

NS16550AN is a plug in replacement for 8250 and 16450 serial chips.  Arrow

Electronics 1-800-932-7769 sells National NS16550AN chips mail order

(credit card) for about $22 each in small quantities.

Chapter 1             Rev 10-19-88  Typeset 10-19-88                     4

Chapter 2                  DSZ Quick Reference                           5

2.  FAIR PLAY

Unlike snazzy menu interfaces, reliable high speed transfer protocols take

many hours of testing, often over expensive phone connections, to develop.

Two decades of communications experience and protocol testing are

distilled in DSZ.

DSZ is a user supported program developed and supported by Chuck Forsberg,

owner of Omen Technology.  Countless hours of work and expen$ive resources

went to develop and support DSZ for the myriad environments users use it

in.  The fact that most of the problems people encounter with DSZ are

caused by other software, hardware problems, failure to read the supplied

documentation, or pirating has not kept users from seeking and getting

help, even if it means calling the author in the middle of his sleep.

Beyond this, the world of communications is constantly evolving, and it

requires a significant ongoing effort to keep DSZ and its documentation up

to date with the current state of the art.  Individuals with little

knowledge of the dynamic nature of telecommunications (or hidden agendas)

have misrepresented the frequency of enhancements to DSZ as a sign that

DSZ is "unstable" and unworthy of "serious consideration".  If you think

such sniping is fair play, you are more than welcome to use one of the

many other protocol programs available, each of which claim to offer speed

and reliability comparable to DSZ.

Non registered users are welcome to use DSZ for a reasonable evaluation

period.  "Reasonable" means "until you've decided whether the program is

worth $20.00".  It does not mean "until some competing program gets around

to adding ZMODEM or True Ymodem(TM)".  Afterwards, ethical use of DSZ

requires registering the copy.

DSZ is not and never has been Public Domain.  Every version of DSZ has

included a Copyright Notice and a prohibition against sale or

modification.  There is no usage counter or other Trojan Horse in DSZ, but

if you use this program, you have a moral and ethical obligation to

register it.

Registration enables OverThruster(TM), YMODEM-g, and other advanced

features.  The registration package serializes all past and future

versions of DSZ.COM and DSZ.EXE.

2.1  Sharing DSZ Files

You are welcome to share unmodified copies of DSZ.ARC with your friends

and associates for their evaluation.  This documentation file and the

MAILER file must be provided with all copies of DSZ except by prior

written permission from Omen Technology Inc.  Because of piracy problems,

Omen Technology must INSIST that DSZ be distributed only as part of the

DSZ.ARC file EXACTLY as provided by Omen Technology Inc or Dynamis

Software.

Chapter 2             Rev 10-19-88  Typeset 10-19-88                     5

Chapter 2                  DSZ Quick Reference                           6

PLEASE!!! Do NOT repack the Omen supplied DSZ.ARC file with Pkarc!!!

The few bytes saved by PKARC are more than offset by confusion and

increased difficulty identifying virus infection and unauthorized patches.

The only approved exceptions to this rule are:

[1]  Dissemination the contents of the DSZ.DOC file (part of DSZ.ARC) in

a convenient machine readable or human readable form, providing the

editorial content is unchanged.

[2]  Encoding the DSZ.ARC file into printable characters for transmission

over text based networks and/or message/news distribution systems.

The "uuencode" program used with Usenet comp.binaries is an instance

of such encoding.

If you distribute DSZEXE.ZOO, DSZ.ARC must be included in the same

directory since DSZEXE.ZOO references files in DSZ.ARC.  The DSZEXE.ZOO

file may not be modified in any way, including repacking with another

archive program.  I ask this favor because Rahul's ZOO is a superior

archiving format, and I want to help it along.

If you see an unpacked, repacked, or otherwise modified DSZ archive or

parts thereof in a bulletin board, please ask the board's operator to

replace such files with DSZ.ARC exactly as provided by Omen Technology

Inc.  Flame them until they behave responsibly.

No version of DSZ may be sold without the prior written permission of Omen

Technology Inc.

Distribution of programs that modify, patch, disassemble, decompile, or

otherwise reverse engineer DSZ without prior written permission by Omen

Technology Inc is a bloody theft of services and a violation of federal

copyright law.  The same applies to debug scripts and documentation

written for the same purposes.  When possible, such piracy will be

referred to the FBI for investigation and prosecution under federal

conspiracy statutes.  Applicable "RICO" law provides for treble damages

and recovery of attorney's fees.  So far, none of the cretins producing

such bilge have seen fit to sign their real name and address to their

attempts at ego gratification.

Distribution of DSZ in any medium without this documentation file is an

unauthorized modification to DSZ that deprives the rightful author of

registration revenues.  That is a theft of intellectual property.  Such

distribution also damages the author also obliges the user and author to

waste time and phone bills scrounging for the documentation, and answering

questions that are fully covered in the documentation.

If any version of DSZ is distributed with any other product, software or

hardware, the product documentation must clearly and prominently state

that the customer's continued use of DSZ requires registration directly

Chapter 2             Rev 10-19-88  Typeset 10-19-88                     6

Chapter 2                  DSZ Quick Reference                           7

with Omen Technology or its duly authorized agent(s).  When DSZ is

distributed on disk, a contemporary copy of ZCOMMDOC.ARC should be

included on the same disk.

Thank you for your support.

2.2  REGISTRATION

After a short evaluation period, ethical users must register their copy of

DSZ with a check for $20.00 in U.S. funds sent to:

Omen Technology Inc.

P.O. Box 4681

Portland, OR 97208

For VISA/MC Registrations, call 503-621-3406

Canadians should remit to:

Dynamis Productivity Software LTD

P. O. Box 5008, Station B

Victoria BC V8R 6N3

The address on your check is sufficient, no need to fill out forms.

Don't bother scrounging for an envelope, just print the INSTANT MAILER

included in this archive!

Users, especially those overseas, may choose to register their copy of DSZ

for $25.00 U.S.  by providing their VISA or Mastercard card number with

expiration date and their name as it appears on their credit card.

Overseas registrations are sent out Air Mail.

Please contact Omen Technology before issuing unfunded purchase orders.

*************************************************************************

The user registration package includes a printed serial number card,

latest versions of the DSZ programs, the installation program which

enables features and bypasses the opening advertisement, and additional

documentation.

The registration package serializes all past and future versions of

DSZ.COM and DSZ.EXE.

*************************************************************************

The unique serial number on the registration card is registered to the

user, and is acceptable proof of registration to corporate auditors.

The putsnp program inserts your unique DSZ Serial Number Password (SNP)

into you copy of DSZ with the following command:

putsnp dsz.com

Putsnp accepts your SNP and asks appropriate questions before inserting

Chapter 2             Rev 10-19-88  Typeset 10-19-88                     7

Chapter 2                  DSZ Quick Reference                           8

the SNP.

Some TSR programs have been known to interfere with putsnp.  If you have a

lockup running putsnp, run it without any TSR programs.

The only legitimate way to disable the opening advertisement in DSZ is to

insert an authorized SNP obtained from Omen Technology into DSZ with the

putsnp program.  Any other suppression or modification of the opening

screen is an illegal, unethical theft of services.

Piracy is stealing.

2.3  Files on Registration Disk

The following files are currently included on the Registration Disk sent

to paid registrants.

DSZEXE.ZOO     Contains DSZ.EXE.

DSZ.ARC        Contains DSZ.COM, this document, DESQview description

files, and an instant no-effort registration mailer.

YZMODEM.ZOO    Contains Official YMODEM and ZMODEM protocol descriptions.

LOOZ.DOC       Documentation on running the LOOZ program

LOOZ.EXE       Program to unpack .ZOO archives

RZSZ.ZOO       ZMODEM C source code, porting base for several dozen ZMODEM

implementations

PUTSNP.EXE     Installs your unique registered serial number into DSZ.

DSZBG.DOC (in DSZBG.ZOO) Background information for DSZ, part of the ZCOMM

User's Manual.  DSZBG.DOC is for paid registered users

only; registered bulletin boards and unregistered users are

welcome to read the ZCOMM manual contained in ZCOMMDOC.ARC.

Chapter 2             Rev 10-19-88  Typeset 10-19-88                     8

Chapter 2                  DSZ Quick Reference                           9

2.4  Bulletin Board Registration

DSZ may be freely used and copied by public bulletin board systems which

agree to the conditions stated in the accompanying bbs_reg.txt file.

Those not abiding to these conditions must obtain a paid registration for

each node.

Free bulletin board registrations are processed in batches.

Chapter 2             Rev 10-19-88  Typeset 10-19-88                     9

Chapter 3                  DSZ Quick Reference                          10

3.  DESCRIPTION

DSZ is a "protocol driver" that uses the ZMODEM, True YMODEM(TM), or

XMODEM protocol to transfer files over a serial port with a variety of

programs.

ZMODEM Transfers feature 32 bit CRC, crash recovery, and advanced file

management.

When calling bulletin boards to download files with ZMODEM you can avoid

the bother of starting DSZ for each transfer by calling DSZ first, before

asking the BBS for the download.          dsz t The "t" command allows you

to keyboard the file request to the BBS.  DSZ's ZMODEM AutoDownload(TM)

then receives the file automatically, without any wasted keystrokes or

delay.  Try it, you'll like it!  (The "t" command is described below.)

YMODEM downloads are almost as easy.  Use the "t" command as shown above,

and hit the F3 key after telling the board to download a file with YMODEM.

ZMODEM and True YMODEM(TM) transfers preserve exact file length and

modification time.

DSZ supports XMODEM-1k (XMODEM with 1024 byte blocks, sometimes mislabeled

YMODEM) and CRC-16 with the sx -k and rc commands.

DSZ is derived from Omen Technology's "Professional-YAM" high reliability

communications tools, (also known as PowerCom(TM)), and supports a subset

of Pro-YAM's file transfer related commands.  This documentation makes

extensive references to contemporary editions of the ZCOMM or Pro-YAM

manual, and is incomplete without it.

DSZ is designed to be called from a communications bulletin board program.

DSZ may also be called from other communications programs, but the

combination lacks much of the enjoyable user friendliness of a program

with integrated ZMODEM.

DSZ uses its own modem I/O.  The original interrupt vector, UART parity,

and interrupt configuration are restored on exit.  The original state of

the 8259 interrupt mask bit for this vector is also restored on exit.

DSZ uses DOS standard output (stdout) for messages.  It never writes

directly to the screen.  DSZ expects its standard output to point to the

console, a logging file, or NUL, not the modem port.  If dsz's standard

output is redirected to the modem (as with ProComm's Host mode DOS shell),

the combination may lock up the computer.  If necessary, dsz's standard

output may be redirected back to the console device with the CON command.

Chapter 3             Rev 10-19-88  Typeset 10-19-88                    10

Chapter 4                  DSZ Quick Reference                          11

4.  EXIT STATUS

DSZ exits with 0 status for successful transfers, or the value of the Q

parameter (default 1) if a file transfer failed.  Some languages and

"door" programs do not return this information to the calling program.

Some languages and "door" programs freak out (hang) on certain non-zero

exit values.

A batch file that calls DSZ directly may be used to detect the DSZ exit

status with the DOS "errorlevel" construct.  Please refer to the Q numeric

parameter below for more information.

DSZ's log file entries may also be examined to determine the success or

failure of a file transfer.

Chapter 4             Rev 10-19-88  Typeset 10-19-88                    11

Chapter 5                  DSZ Quick Reference                          12

5.  COMMANDS

dsz [CON] [port N] [speed S] [handshake XX] [D] [d] command

DSZ commands are an improper subset of Pro-YAM/PowerCom/ZCOMM commands.

They are fully described in the ZCOMM User's Manual, contained in

ZCOMMDOC.ARC.  Portions of that document are included with the DSZ

registration disk.  Please refer to that document for details on the

following commands.

***********************************************************

NOTE WELL

Except as noted, all commands must be in entered lower case.  (Case is

significant!)

***********************************************************

When used (in any combination), the CON, port, speed, handshake, and d

commands must be given in the order shown.

CON   Redirect message output to the DOS "CON" (console) device,

overriding any previous redirections.  This is necessary to avoid

lockups caused by Procomm host mode.  Most programs do not require

this hack.  This command is unique to DSZ.

estimate length [speed] Use the specified length and optional specified

effective speed for the next receive file to be created.  Please

refer to the ZCOMM manual for details.

handshake XX Enable one of several choices of hardware handshake.  (The

default is XOFF/XON software handshake).  This command is documented

in the ZCOMM or Pro-YAM manual, and in DSZBG.DOC.  (The ZCOMM manual

is available on all bulletin boards registered for DSZ as

ZCOMMDOC.ARC.) On DSZ the "handshake" command may be abbreviated to

"ha".

port N select port N (n = 1,2,3,4,5,6,7,8, 10,11,12,13,14,15,16,17,18)

Default is port 1 (COM1) or read from the DSZPORT dos environment

variable.  COM10 to COM18 refer to the COM1 to COM8 ports on the

PS/2.

In multitasking operation, accessing a port used by another running

program may hang the machine.

EXAMPLE: to use COM2:

dsz port 2 rz

portx ... Select arbitrary port address and interrupt vector.  Please

refer to the ZCOMM or Professional-YAM manual for information on

this command.

Chapter 5             Rev 10-19-88  Typeset 10-19-88                    12

Chapter 5                  DSZ Quick Reference                          13

speed S Set the specified speed.  This command is rarely needed because

DSZ reads the port's current speed by default.  Please refer to the

FLOW CONTROL, MAIN COMMANDS and SOFTWARE COEXISTENCE chapters in the

ZCOMM or Pro-YAM manual for important information about the speed

command.

D     Disables the modem (drops Data Terminal Ready, DTR) if carrier is

lost.  This may be used to prevent spurious events (phone ringing)

from interfering with the calling bulletin board's recovery from

carrier loss.

d     Disables carrier dropout monitoring, for modems that do not properly

drive the "Carrier Detect" input on the PC.  If you see a No Carrier

Detect message, you need to give the d command.

LARG  Convert ARG to lower case, then parse arg.

pxN   Set a numeric parameter x to N.  These parameters are mostly a

subset of the Pro-YAM/ZCOMM numeric parameters, and are listed

below.  They assume their defaults each time DSZ is called.  Without

an argument, p displays the numeric parameters and their values.

EXAMPLE: C>dsz p

z pxN Set zmodem numeric parameter x to N.  These parameters

correspond to the Pro-YAM or ZCOMM "zmodem" parameters and rarely

need changing.  These parameters assume their defaults each time DSZ

is called.  Without an argument, z p displays the zmodem numeric

parameters and their values.

restrict Restrict pathnames to the current disk and directory tree, and

disallow overwriting of existing files.  When DSZ is restricted, it

will refuse to transfer files containing the string autoexec.bat and

command.com in upper or lower case.  This provides some defense from

malicious uploaded files.

t     [-8g] Talk with ZMODEM AutoDownload(TM).  The optional -8g

(available on registered copies only) allows "BBS (ANSI) graphics".

(A video device driver such as ANSI.SYS must be loaded to suppport

some types of graphics.) A small subset of ZCOMM/Pro-YAM term

function subcommands are recognized.  Keyboard F1 or ALT-X to exit.

F3 receives file(s) with True YMODEM(TM), ALT-P toggles parity

between the 8n default and 8g (enables ANSI "graphics").  ALT-H

toggles two local echo ("Half Duplex") modes; the second displays a

newline (CR/LF) when Enter is keyboarded.  Keyboarded ASCII

characters are sent to the modem.

Modem characters print on the screen.  A device driver such as

ANSI.SYS may be used to recognize escape sequences.

DSZ's ZMODEM AutoDownload(TM) detects the start of a ZMODEM file

Chapter 5             Rev 10-19-88  Typeset 10-19-88                    13

Chapter 5                  DSZ Quick Reference                          14

download and accepts the file(s) without operator intervention.

rz    [-abenprt+Yy] [dir] [file ...]          (ZMODEM receive)

rb    [-abt+y] [dir] [file ...]     (YMODEM receive)

rb -g [-abt+y] [dir] [file ...]     (YMODEM-g receive)

rc    [-abt+y] file1                (XMODEM/CRC receive)

ro    [-abt+y] file1                (XMODEM OverThruster receive)

rx    [-abt+y] file1                (XMODEM receive)

rx    -g [-abt+y] file1             (Qmodem-G receive)

sz    [-abefnprtY+y] [-w[N]]  file ...        (ZMODEM send)

sb    [-ft] [PREFIX=p | ONAME=n] file ...     (YMODEM, YMODEM-g send)

sb -k [-ft] [PREFIX=p | ONAME=n] file ...     (YMODEM-1k send)

sx    [-t] file1                    (XMODEM, XMODEM/CRC, Qmodem-G send)

sx -k [-t] file1                    (XMODEM-1k send)

The meanings of the optional arguments are described in the ZCOMM manual.

Send files may be specified indirectly:

dsz sz @C:files.lst

sends the files in C:files.lst.  This provides another means of bypassing

the DOS command line limit.  The first letter must be "@" and the third

letter must be ":" to distinguish this from an ordinary pathname.

Pathnames in the indirect may be separated with any combination of

space(s), tab(s), CR and/or LF.  No spaces may appear inside a pathname;

"FOO     .BAR" won't work.

5.1  Interrupt Keys

Ctrl-Break Terminates the transfer in progress.

ALT-N Abruptly terminates (NUKES) the transfer in progress.

Ctrl-X Typing a dozen Ctrl-X characters to another ZMODEM program

terminates ZMODEM operations.

ALT-F Skips the current file in ZMODEM transfers.

ALT-B Cancels the current ZMODEM session.

Chapter 5             Rev 10-19-88  Typeset 10-19-88                    14

Chapter 6                  DSZ Quick Reference                          15

6.  NUMERIC PARAMETERS

DSZ uses the following main numeric parameters, settable with the pxN

command.

EXAMPLE: C>dsz pQ0

In addition, DSZ uses the Pro-YAM/ZCOMM zmodem numeric parameters

described in the Pro-YAM/ZCOMM reference manual.

B   Set the size of buffer used for disk reads and writes.  The default is

1024, 8192 maximum.  If your copy of DSZ is running on a floppy disk

system or a very slow turkey hard disk, a larger value may give better

results.  Too large a value may exhaust memory, induce receiver

timeouts, cause disconnects(!), or generate spurious CRC errors.  If

performance logging doesn't work, this parameter is too big.  This

parameter is supported on versions compiled with TurboC.

EXAMPLE: C>dsz pB4096 sz A:floppy.fil

Setting the buffer size larger may lock out the performance log,

DSZFILES, and/or indirect pathname list in the "tiny model" DSZ

flavor.  If you aren't bloody sure you need a bigger buffer, don't use

one.

O   This parameter controls XMODEM OverThruster(TM).  It is described in

dszbg.doc.  This file is part of the software package sent to paid

registrants.

Q   Error code returned to DOS (default 1).  (Success always returns 0.)

Some programs toss cookies when confronted with certain exit codes.

EXAMPLE: C>dsz pQ-1 sz foo.bar

S   Timeout in seconds (default 15) waiting for restraint release (XON

and/or handshake).

W   Wait for Enter key before exiting after an error.  Effective on

registered copies.  Useful when calling DSZ from DESQview, allows the

window to close unless there is an error message that should remain

visible.

a   Calibrates certain protocol timeouts used with XMODEM and YMODEM

transfers (Cybernetic Data Recovery(TM)).  This parameter does not

affect ZMODEM transfers.

The a numeric parameter is automatically set to 2000 for a 4.77 mHz PC

or XT, 4000 for a PS2/30, 6000 for an 8 mHz 1ws AT, and 12000 for a

Chapter 6             Rev 10-19-88  Typeset 10-19-88                    15

Chapter 6                  DSZ Quick Reference                          16

PS2/80.  If your computer has a speedup card or otherwise runs

unusually fast (or slow!) for its type, you may set the a numeric

parameter to a value that reflects its speed.

EXAMPLE: dsz port 2 pa4000 rb

NOTE: Setting this parameter does not suppress the "Unknown Machine

Type" message which is displayed before the command line is scanned.

d   A non zero value (the default) causes incoming files received with

ZMODEM and full YMODEM transfers to have their Date set to that sent

in the file header.  ZMODEM's file management features depend on the

transmission of each file's modification date.  Files will have the

same creation date, even if the two machines are in different time

zones.  When transferring files to/from a Unix system, creation dates

are interpreted according to GMT or Universal Coordinated Time.  File

dating may be disabled with a pd0 command.

NOTE: Backdating file creation times may confuse some backup programs.

v   Verbose level (debugging output) Negative values inhibit some routine

messages.  Setting v to 1 or greater causes serialized copies to ring

the bell when a transfer is done.

z   Timezone in minutes behind GMT, used for ZMODEM and True YMODEM file

transfers to coordinate the actual date of the file with computers in

other time zones.  Effective on registered copies.  PDT or MST

corresponds to 420.  This may be preset with the ZONE DOS environment

variable.

Chapter 6             Rev 10-19-88  Typeset 10-19-88                    16

Chapter 7                  DSZ Quick Reference                          17

7.  ENVIRONMENT VARIABLES

The following DOS environment variables may be set to modify DSZ

operation.

DIRRX (Registered copies only.) Specifies the directory to receive

(download) files.  If the sting in DIRRX ends with "/", directory

and/or disk designators in the incoming pathname are stripped, and

only the stem (name and extension) is used.  Otherwise, if an incoming

file has an absolute pathname, the incoming pathname is used.

EXAMPLE: C>set DIRRX=C:/rxdirect/

C>dsz rz -r

DSZFILES May contain a list of up to 9999 pathnames to transmit, with

entries separated by spaces.  If present, DSZFILES is used in place of

a normal command line list of files to send with the sb and sz

commands.

EXAMPLE: C>set DSZFILES=file1 file2 file3 *.c

C>dsz port 2 handshake on sz -r

In practice, this string would be built up by a calling program and

passed to DSZ with a "spawnvpe", "execvpe", or similar subroutine

call.

SEE ALSO: Pathname list specification @C:files.lst

DSZLOG log file to receive entries for files sent and received.  (Default

null).  Sample entries:

Z  46532 38400 bps 3324 cps   0 errors    66 1024 DSZ.COM 1177

Z 124087 19200 bps 1880 cps   0 errors     6 1024 MXY.TMP 1177

Transfer times are based on the DOS time of day clock and have one

second jitter.  As with all protocol throughput reports generated by

Omen Technology programs (and contrary to the false claims of some

critics), DSZ's throughput figures for ZMODEM, Kermit, and True

YMODEM(TM) transfers do not take "credit" for pathname and/or protocol

overhead characters.  DSZ's throughput reports do not include the time

required to send the pathname.  This understates the true time

required to transfer files, but the time required to send the pathname

often depends on the alacrity of the operator, and thus does not

always reflect the performance of the protocol itself.

Transfers measuring less than one second are reported as having 9999

characters per second throughput.

The unlabeled numeric field after the error (retry) count is the

number of flow control stoppages encountered sending the file.  This

Chapter 7             Rev 10-19-88  Typeset 10-19-88                    17

Chapter 7                  DSZ Quick Reference                          18

number gives an indication of flow control restrictions (overloaded

networks, etc.) encountered during the transfer.

The last numeric field before the file name is the block or subpacket

length used for the last block in the file.

The last field is the serial number of the other program, or -1 if

this information is not available.

The code letter indicates the type of transfer, Z (ZMODEM), S

(X/YMODEM), R (X/YMODEM), E (error), or L (transfer terminated by loss

of data carrier).

WARNING: If DSZ encounters a loss of carrier or other fatal error

condition before a file commences transmission, a log entry will not

be made.

EXAMPLE: C>set DSZLOG=C:/yam/dszlog

DSZOPT (Registered copies only) String containing default options for DSZ

file transfers.  Options are optional modifiers to the DSZ sz, rz, sx,

rx, sb, rb commands.

An 8n may be appended to the string to make BBS (ANSI) graphics the

default for DSZ's t command.  The ability to preset these options is

especially helpful for controlling ZMODEM automatic downloads from

DSZ's t command.

EXAMPLE: C>set DSZOPT=r

Enables DSZ's Intelligent Crash Recovery(TM) as a default.

Let's say you wish ZMODEM crash recovery, and BBS "graphics" for the

DSZ term function:

C>set DSZOPT=r8g

WARNING: Do NOT include "ena" or any other commands in the DSZOPT

string.  With the exception of the "8g" mode, only **options** may be

placed in the DSZOPT string!  (Options are the letters that modify the

sz, rz, sb, etc. commands.  In "rz -r", "r" is an option.)

MAXBYTES Maximum number of bytes allowed for transfer.  If this number is

exceeded, DSZ skips the offending files (ZMODEM) or aborts remaining

files in the batch (YMODEM).

DSZPORT Default communications port override, default 1.

EXAMPLE:

C>set DSZPORT=2

Chapter 7             Rev 10-19-88  Typeset 10-19-88                    18

Chapter 7                  DSZ Quick Reference                          19

ZONE sets the z numeric parameter to the number of minutes the current

local time lags GMT time.  PDT or MST corresponds to 420.  (Registered

copies only)

EXAMPLE:

C>set ZONE=420

Chapter 7             Rev 10-19-88  Typeset 10-19-88                    19

Chapter 8                  DSZ Quick Reference                          20

8.  EXAMPLES

dsz sz c:foo.* b:*.c          (send files)

dsz sz -r yzmodem.arc         (CRASH RECOVERY: resume interrupted

transfer)

dsz rz -r                     (CRASH RECOVERY: resume interrupted

transfer)

dsz handshake both sz yzmodem.arc       (Hardware+Software handshake)

dsz sz -n *.*                 (send files that are newer or rx has no

copy)

dsz sz -Yn *.*                (Only send files that are newer than

receiver's copy)

(Do not send files that do not exist on

receiver's disk)

dsz rz C:/UPLOADS             (receive files to specified directory)

dsz port 2 speed 38400 restrict rz

dsz rz  -y prog.arc           (ZMODEM, single file pathname override)

dsz z pr1 rx foo.dat          (XMODEM receive, remove file if error)

dsz pO60 ro foo.dat           (XMODEM/OverThruster receive, GEnie MNP)

DSZ LRC L-Y NEWPROG.ARC       (generate lower case "rc -y" command)

dsz pv2 sz foo >debug.dmp     (Debugging data dump)

dsz sz @C:/bbs/files.lst      (Indirect file list)

Chapter 8             Rev 10-19-88  Typeset 10-19-88                    20

Chapter 9                  DSZ Quick Reference                          21

9.  BACKGROUND OPERATION

Three DESQview description files are provided to demonstrate background

operation with modest (66 kb window) memory requirements.  DSZ uses tight

C/ASM code, table driven CRC generators, and buffered receive and transmit

for good performance in the background without hogging CPU cycles.  On a

QIC Labs PC-AT clone, DSZ can download files in the background (3

background clock ticks, 9 foreground clock ticks) with a throughput better

than 9600 bps.

2S   Prompts for file name(s) and sends them with ZMODEM protocol.  File:

2S-PIF.DVP

2T   Talks to the modem and allows manual YMODEM (F3) or automatic ZMODEM

downloads.  File: 2T-PIF.DVP

2A   Like 2T, but using Quarterdeck's dvansi.com memory resident ANSI CRT

device driver to process ANSI screen codes.  File: 2A-PIF.DVP DS.BAT

is the batch file used by 2A.

These files are written for a port 2 local connection and \TMP directory,

but may be modified to suit your requirements using the DESQview CP

command.  The W numeric allows DESQview to close the window after DSZ

finishes, providing a pleasant user interface.

Operation at high speed (above 9600 bps) requires the DESQview "Optimize

communications" performance Advanced Setup option.

***********************************************************

WARNING

When running DSZ under a multi tasking system such as DESQview, do not

allow two programs to access the same comm port at the same time.  At

best, such actions will disrupt communications.  At worst, your computer

will attempt to open a subspace channel to V'ger.

***********************************************************

Chapter 9             Rev 10-19-88  Typeset 10-19-88                    21

Chapter 10                 DSZ Quick Reference                          22

10.  CALLING DSZ FROM COMM PROGRAMS

It may be necessary to give a

SET COMSPEC=A:\COMMAND.COM

(change as needed) in your AUTOEXEC.BAT file to allow programs to call

DSZ.

10.1  GT POWER 13.00/14.00

GT Power version 13/14 includes a YMRX.BAT (YMODEM-g receive) file with an

"rz" command.  You guessed it, "rz" is for ZMODEM protocol (the z means

ZMODEM!).  Please change the file to use the correct "rb -g" for YMODEM-g.

The files ZMTX.ARC and GT1300.NOT available on many bulletin boards

contain useful information for GT Power users.

10.2  PROCOMM

DSZ may not operate properly when called from Procomm if Procomm has files

open at the time.

When running under ProComm's Host mode, failure to use the CON command may

cause Procomm to lock up the computer.

The POE, PCALL, DSZ_ZAP, and PROZ programs have been popular in

interfacing DSZ to Procomm's editor gateway.

Hint: the DOS PATH environment variable must include Procomm's directory

to allow POE to locate all the POE files,.  The POE files should be stored

in the Procomm directory.  Setting the "PROCOMM" environment variable is

insufficient.

10.3  PROCOMM PLUS

DSZ may not operate properly when called from Procomm if Procomm has files

open at the time.

Here are some handy settings for Procomm Plus.

First, set the DOS environment as necessary before running Procomm:

SET DSZPORT=2                           (Only needed for COM2)

SET DSZLOG=DSZ.LOG            (If you wish a log file)

Then set Procomm's external protocol strings:

A- External protocol 1 upload filename  ...ZMODEMU

B- External protocol 1 download filename ... ZMODEMD

And then some short batch files:

ZMODEMU.BAT:                  DSZ sz %1 %2 %3 %4 %5 %6 %7 %8 %9

Chapter 10            Rev 10-19-88  Typeset 10-19-88                    22

Chapter 10                 DSZ Quick Reference                          23

ZMODEMD.BAT:    DSZ rz

To make downloads easier, use the "dsz t" command described above.

ZMODEMT.BAT:                  DSZ t

10.4  Qmodem

The following is based on a message from Mike Cody, sysop of HomePlate

313-385-9969 3/12/24/96 HST

Go into Qinstall and go to frame 3 Directories. Then select External

protocols #3 I think. Now you will see a definition area to tell Qmodem

the Letter,batchfile combo to use.

Now put Z in the letter definition, Zup.bat in the u/l batch zdown.bat in

the d/l batch, and Y in the btch protocol. Now save it at the menu with

f10 and S.

Then go to DOS and create the following batch files in your Qmodem

directory...

Zdown.bat: DSZ port %2 speed %1 rz -w %3

Zup.bat: DSZ port %2 speed %1 sz %3

These are the batch files that will be called by Qmodem.  If uou did

everthing right when uoy hit PGDN or PGUP you will see an external

protocol called Z) Zmodem in the box. For an easier time do a serch for

either External.arc or maybe the string "Qmodem" in the file database as

there is a file with several external prootcols and batch files available

in the Mahoney file section (EXEC BBS).

10.5  BULLETIN BOARDS

When installing DSZ on a bulletin board or host system, be sure to check

for proper operation when the caller hangs up while a file is being sent

or received.

Currently, Only the TurboC version supports file sharing (shared read

access on files sent by  dsz).

Some bulletin board interface routines do not call DSZ to send a file

until they have verified the file's presence by opening the file.  If this

checking is done in DOS's default exclusive open mode, dsz's file sharing

will be defeated.

Chapter 10            Rev 10-19-88  Typeset 10-19-88                    23

Chapter 11                 DSZ Quick Reference                          24

11.  FLOW CONTROL

If you plan to send files over Telenet (PC-Pursuit), you must refer to the

Pro-YAM or ZCOMM Reference manual (in ZCOMMDOC.ARC) FLOW CONTROL chapter

for vital information on flow control considerations necessary for proper

operation of ZMODEM and other streaming protocols.

Chapter 11            Rev 10-19-88  Typeset 10-19-88                    24

Chapter 12                 DSZ Quick Reference                          25

12.  ERROR MESSAGES

COM2 1200 bps Carrier Detect enabled Handshake slow

Command line = (commands)

This is a normal message displayed to help resolve problems with incorrect

batch files.  The port number, speed, carrier detect ("enabled" or

"DISABLED") and "handshake" indicate the selected mode of operation.

Unknown Machine Type XX, 'a' parameter preset to 6000 indicates a ROM BIOS

that does not follow the IBM specification for identifying the machine

type.  If your computer causes DSZ to display this message, you may set

the a numeric parameter described elsewhere in this document.  This

message is generated before any commands are read, so do not be alarmed

when setting the a parameter doesn't make this message go away.  The a

parameter does not affect ZMODEM transfers.

Need more memory to run DSZ or Abnormal program termination is displayed

by the TurboC startup routine if DSZ is run in too small a memory

partition.  If you see this message, use a larger memory partition for DSZ

or remove some memory resident programs.  This message may also appear if

the calling program fails to free memory before calling DSZ.

No Carrier Detect is displayed when DSZ recognizes a loss or lack or

carrier detect signal from the modem.  This is caused by a line

disconnect, defective cable, or incorrect modem strapping.  It may be

disabled with the d command.  Sometimes this is caused by setting the B

parameter to too large a value.

Garbage Count Exceeded Indicates ZMODEM has not detected a valid start of

header in a reasonable number of characters.

If DSZ becomes "stuck" on emitting this messages, it is possible the

sending program may not be able to recognize and act upon transmission

requests (ZROPS headers).  The sending program should be corrected.  In

the meantime, setting a ZMODEM packet length with the "zmodem pl4096"

command forces the sender to wait for the receivers' progress reports.

Permission denied comes from the Microsoft C Library.  This may be caused

by attempts to modify a read only file or directory, or create a file in a

nonexistient directory.

A furtive flashing appearance of XOFF or FLOW during ZMODEM or YMODEM-g

file sending signifies DSZ is responding to flow control restraint.  This

is normal operation when buffered modems or packet switched networks are

used.  ZMODEM's robustness and reliability allow creative users to

transfer files over unusual multiple computer lash-ups, causing modem data

lights and computer status displays to dance in unfamiliar rhythms.  Got

ZRPOS indicates the receiving program has detected a transmission error

and has requested retransmission.  In other words, ZMODEM is doing its job

of detecting and correcting missing and garbled data.  If this happens

more often than observed line errors would indicate is reasonable, there

Chapter 12            Rev 10-19-88  Typeset 10-19-88                    25

Chapter 12                 DSZ Quick Reference                          26

may be a flow control problem in one of four areas:

+ Between the sending program and its modem.  If using an interface

speed higher than the transmission speed, or modem to modem error

correction (MNP, LAPB, PEP, etc.), check that the modem and the

sending program are configured for the same flow control methods.  DSZ

is faster than most other programs, and may expose flow control

problems not seen with slower software.

+ Between the sending modem and the network.  Correcting this requires

"handshake both" to allow the sending program to honor both hardware

(CTS) flow control from the modem and software flow control (XON/XOFF)

from the network.

+ Between parts of the network.

+ Between the receiving modem and the receiving program.  This can

happen at high transmission speeds when the receiver has specified a

large I/O buffer size with the B numeric parameter.

Excessive interrupt latency from extended memory ramdisks or disk

caches, and poorly written hard disk drivers also wreak havoc on

streaming protocols but do not affect start-stop protocols (XMODEM,

etc.).

To correct these problems, give the "handshake slow" command to the

receiver.

Byte NNNN retry M: <error message> These messages appear when the protocol

has detected an error and is attempting to retransmit the garbled data.

These are normal messages.

Serial Input Error: Line Status Register HH indicates a slow device

driver, TSR program, or other software is disabling interrupts too long

for DSZ to accept incoming data, and some characters are lost.  A slow

response to folow control also triggers this message.

The value of the line status register is displayed in HEX.  The following

error conditions when present OR into the Line Status Register.

01   Interrupt level circular buffer overrun.  The remote did not respond

to 's XOFF or hardware flow control.

02   Data Overrun generally caused by formware or other software locking

out interrupts for excessive periods.  Other chapters in this

document give suggestions on decreasing excessive interrupt latency.

Chapter 12            Rev 10-19-88  Typeset 10-19-88                    26

Chapter 12                 DSZ Quick Reference                          27

08   Framing Error generally caused by line noise or an incorrect

transmission speed or format (number of bits).  Most protocols

require 8 bits no parity for proper operation.  This is set

automatically by Omen Technology programs, but other programs may not

be so smart.

10   Break Interrupt generally caused by line noise or an incorrect

transmission speed or format (number of bits).

Please refer to the In Case of Difficulty chapter of this document for

solutions.

Less commonly encountered DSZ error messages are explained in the Pro-YAM

manual.

Chapter 12            Rev 10-19-88  Typeset 10-19-88                    27

Chapter 13                 DSZ Quick Reference                          28

13.  ASSOCIATED DOCUMENTS

Consult the Professional-YAM, PowerCom, or ZCOMM manual chapter "OPTIONS

FOR FILE TRANSFERS" for descriptions of the file transfer options.  The

"FILE TRANSFER BASICS" chapter explains the relative merits of XMODEM,

YMODEM, ZMODEM, and other protocols and gives tips on their use.  The

"MAIN COMMANDS" chapter describes the rb, rx, rc, rz, sb, and sz commands.

The "MODEM INTERFACING" and "FLOW CONTROL" chapters contain important

information on the use of buffered modems with DSZ.

Information on the ZMODEM Inter Application File Transfer Protocol may be

found in ZMODEM.DOC, part of YZMODEM.ZOO (yzmodem.arc).

C Source code for XMODEM, YMODEM, and ZMODEM transfers is included in

RZSZ.ZOO.  This file includes a Pro-YAM/ZCOMM script to upload a bootstrap

file to a Unix system, force its compilation, and (if the compilation is

successful) upload the rz/sz source files.

The C source code now includes VAX/VMS support.

Chapter 13            Rev 10-19-88  Typeset 10-19-88                    28

Chapter 14                 DSZ Quick Reference                          29

14.  FILES

2S-PIF.DVP, 2A-PIF.DVP, DS.BAT, 2T-PIF.DVP DSZ.COM, and DSZ.DOC constitute

Omen Technology's DSZ.ARC.

2S-PIF.DVP, 2A-PIF.DVP, DS.BAT, and 2T-PIF.DVP are used with the DESQview

multitasking executive for background operation.

ZCOMMDOC.ARC (manual), ZCOMMEXE.ARC (programs), ZCOMMHLP.ARC (help

processor and database), ZCOMM communications tools.  The ZCOMM files

should be available on the BBS you downloaded DSZ.ARC from.  If these

files are not available, the BBS is not fulfilling the concractual

requirements of free BBS registration of DSZ.

YZMODEM.ARC contains XMODEM, YMODEM, and ZMODEM Protocol Descriptions.

RZSZ.ZOO contains ZMODEM "C" source code.  (COMPLETE VAX VMS and

Unix/Xenix programs).

DSZ.EXE compiled with the Xenix to DOS XC provides moderately higher

maximum throughput (1700 vs 1300 cps downloading to a 4.77 mHz XT).  The

speed difference is a function of the compiler, the source code is the

same.  The Xenix XC version requires more memory, and does not support

file sharing or a large I/O buffer.  This version is available in

DSZEXE.ZOO.

ZCOMM and Pro-YAM also run at a higher speed than DSZ.COM.

DSZBG.ZOO contains DSZBG.DOC.  Part of the DSZ registration disk (and not

for public distribution), this file provides background information on the

ZCOMM/Pro-YAM commands provided in DSZ.

LOOZ.EXE unpacks .ZOO archives.  LOOZ.DOC describes LOOZ.EXE.  The ZOO

archive format is a machine independent improvement on the PC-DOS specific

.ARC format.

Chapter 14            Rev 10-19-88  Typeset 10-19-88                    29

Chapter 15                 DSZ Quick Reference                          30

15.  CHANGES

Be sure to save your serial number/password (SNP) card and the putsnp

program you received from Omen to use with future versions of DSZ you may

download!

10-19-88  ZMODEM crash recovery of files where the source is shorter than

the destination is now (properly) inhibited.

The "z" parameters may now be set in the context of an XMODEM or YMODEM

transfer.

8-30-88 Bloated file transfer times when spanning midnight have been

corrected.  Some of the error messages have been made more understandable,

and are explained in this document.

8-10-88, 7-19-88 Further Documentation enhancements.

7-11-88  Documentation enhancements.  Putsnp program now compiled under

TurboC, resets console raw mode to avoid lockup with off the wall device

drivers.

Portx command allows selection of comm port with specified base address

and interrupt vector.

YMODEM-g modified for better compatibility with non standard programs such

as EXEC-BBS.

6-07-88 Minor changes which do not affect normal file transfers.  The

speed/handshake status readout now reflects the conditions used for the

actual transfer instead of the first speed or handshake command

encountered.

With the sz command, the k option forces an initial 1024 byte subpacket

length.  This is for you folks that just can't wait until Dsz's adaptive

optimizations get up to 1k subpackets.

EXAMPLE: sz -k *.c *.h

REMEMBER: ZMODEM subpacket length isn't the same as the difference between

128 and 1024 byte XMODEM blocks!!!

DSZ.COM is now compiled with TurboC under SCO 386 Xenix+VP/ix to reduce

the possibility of virus infection.

5-7-88  A spurious message about Crash Recovery when the -r option is used

and no old file is present has been suppressed.  "Handshake slow" is more

effective than previously.

4-23-88  Good news for tired frog stompers!  The ZMODEM rz/sz programs are

Chapter 15            Rev 10-19-88  Typeset 10-19-88                    30

Chapter 15                 DSZ Quick Reference                          31

now available in a VMS C flavor.  The VAX/VMS source files are contained

in the RZSZ.ARC (RZSZ.ZOO) archive file.

Support for CP/M format (0x1A EOF character) log files has been restored.

(Previously, it had been removed to save memory).

The value of the DSZOPT string (if set) is now displayed to aid debugging.

A coding error that caused excruciatingly slow throughput if a

transmission error or common section mismatch was detected *after* the

sending program had reached EOF was fixed.

"Handshake slow" has been added to disable the RTS signal to the modem

while writing to disk, even if the interrupt buffer is not chock full of

bytes.  This should help systems with excessive interrupt latency on disk

writes.

4-14-88 The DSZOPT=8g feature (for the t command term function) now works

as documented.

Bogus file count displays seen on YMODEM and ZMODEM downloads have been

fixed.  (They were caused by the serial number being sent in decimal

instead of octal.)

4-6-88 Send pathnames may be specified indirectly:

dsz sz @C:files.lst

sends the files in C:files.lst.  This provides another means of bypassing

the DOS command line limit.  The first letter must be "@" and the third

letter must be ":" to distinguish this from an ordinary pathname.

Obligations of Bulletin Boards requesting free BBS registrations have been

amended.  Unethical behavior on the part of a small minority prompted

these clarifications.

3-29-88 There never was a 3-26-88 version, except for use by the

Sandinista troops in Honduras.

3-26-88 First general release compiled with TurboC version 1.5.

Outstanding meganit: on certain networked systems, the reported number of

files on batch send is ridiculous.

This version no longer needs or accepts the "memory allocation patch".

This version has a number of changes to ease interfacing and problem

resolution.  The command line and acive com port are displayed to aid

debugging of batch files that call DSZ.

The DIRRX environment variable may be used to specify a directory for

receiving files.  If the DIRRX environment variable ends with a slash, any

drive designators and/or directory names in the incoming file are

stripped.

Chapter 15            Rev 10-19-88  Typeset 10-19-88                    31

Chapter 15                 DSZ Quick Reference                          32

The DSZFILES environment variable may be used to specify an alternate list

of pathnames to transmit whose length is not limited by the DOS command

line.  It is typically used by calling programs using one of the C library

"spawn" functions.

Setting the v (verbose) parameter to 1 or greater causes serialized copies

to ring the bell when a transfer is done.

2-28-88 More changes in DSZ.DOC, no changes to the software.

2-8-88 DSZ.DOC has been extensively rewritten and reorganized.  DSZ now

supports the advanced buffering of the National NS16550AN and Intel 82510

serial interface devices.  Arrow Electronics 1-800-932-7769 sells National

NS16550AN chips mail order (credit card) for about $22 each in small

quantities.

ZMODEM subpacket length adjustment has been improved to deal with Telenet

congestion.

Pirated/hacked versions of DSZ have cost users and DSZ's developer

thousands of dollars in lost time and other expenses, in addition to

injuring the reputation of Omen Technology and its products.  Regrettably,

Omen Technology cannot support prior versions of DSZ because of this

piracy.

Name             Length                           Date        Time   CRC

dsz.com          50582  Squeezed   10%     45772   7 Jun 88   3:57a  5fa8

dsz.com          50704  Squeezed   10%     45870  11 Jul 88   7:51a  1036

dsz.com          50736  Squeezed   10%     45863  30 Aug 88   4:04a  aae1

dsz.com          50784  Squeezed   10%     45968  19 Oct 88  12:07a  f0fe

Your copy of DSZ should show this information as provided by

arc v dsz.arc

or

pkxarc -v dsz.arc

If your copy shows a different CRC or file length, insist on a legitimate

copy and report the source of the bogus/pirated copy to Omen Technology

Inc.

Chapter 15            Rev 10-19-88  Typeset 10-19-88                    32

Chapter 16                 DSZ Quick Reference                          33

16.  Other Omen Technology Products

Omen Technolgy makes Professional-YAM, a comphrensive comms program with

unmatched power.  Flavors are available for PC-DOS, Unix, Xenix, and OS/2

Protected mode.

ZCOMM is a full featured shareware version with a $40.00 basic

registration.

Chapter 16            Rev 10-19-88  Typeset 10-19-88                    33

Chapter 17                 DSZ Quick Reference                          34

17.  ACKNOWLEDGEMENTS

32 bit CRC code courtesy Gary S. Brown.

The High Reliability Software(TM), TurboLearn Script Writer(TM),

Cybernetic Data Recovery(TM), AutoDownload(TM), Intelligent Crash

Recovery(TM), Error Containment(TM), Full Time Capture(TM), True

YMODEM(TM), OverThruster(TM), Password Guardian(TM), CryptoScript(TM), and

TurboDial(TM) are Omen Technology trademarks.

PowerCom was trademarked in 1985 by POLYTRON Corporation, Hillsboro

Oregon.

DSZ and this documentation are Copyright 1988 by Omen Technology Inc, all

rights reserved.  True YMODEM(TM) is an Omen Technology Trademark

signifying YMODEM protocol support that meets the full YMODEM

specification.

Omen Technology Incorporated

The High Reliability Software

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

Modem: 503-621-3746 Speed 1200,2400,19200 PEP

Compuserve:70007,2304  GEnie:CAF

Chapter 17            Rev 10-19-88  Typeset 10-19-88                    34

CONTENTS

1.  IN CASE OF DIFFICULTY............................................   2

1.1   Incorrect Batch Files......................................   2

1.2   No Carrier Detect..........................................   2

1.3   PKARC 3.6..................................................   2

1.4   COMMAND NOT FOUND..........................................   3

1.5   DOS 3.x....................................................   3

1.6   Lockups....................................................   3

1.7   IBM PS/2 Model 50Z.........................................   3

1.8   Brain Damaged UARTS........................................   3

1.9   Weird Iron.................................................   4

1.10  Will the real YMODEM please stand up?......................   4

1.11  286 and 386 Extended Memory................................   4

2.  FAIR PLAY........................................................   5

2.1   Sharing DSZ Files..........................................   5

2.2   REGISTRATION...............................................   7

2.3   Files on Registration Disk.................................   8

2.4   Bulletin Board Registration................................   9

3.  DESCRIPTION......................................................  10

4.  EXIT STATUS......................................................  11

5.  COMMANDS.........................................................  12

5.1   Interrupt Keys.............................................  14

6.  NUMERIC PARAMETERS...............................................  15

7.  ENVIRONMENT VARIABLES............................................  17

8.  EXAMPLES.........................................................  20

9.  BACKGROUND OPERATION.............................................  21

10.  CALLING DSZ FROM COMM PROGRAMS...................................  22

10.1  GT POWER 13.00/14.00.......................................  22

10.2  PROCOMM....................................................  22

10.3  PROCOMM PLUS...............................................  22

10.4  Qmodem.....................................................  23

10.5  BULLETIN BOARDS............................................  23

11.  FLOW CONTROL.....................................................  24

12.  ERROR MESSAGES...................................................  25

13.  ASSOCIATED DOCUMENTS.............................................  28

14.  FILES............................................................  29

- i -

15.  CHANGES..........................................................  30

16.  Other Omen Technology Products...................................  33

17.  ACKNOWLEDGEMENTS.................................................  34

- ii -

DSZ - a ZMODEM, True YMODEM(TM), XMODEM File Transfer Program

Chuck Forsberg

Omen Technology Inc

ABSTRACT

ZMODEM - Not just another XMODEM hack

The ZMODEM file transfer protocol provides reliable file and command

transfers with complete END-TO-END data integrity between application

programs.  ZMODEM's 32 bit CRC protects against errors that continue to

sneak into even the most advanced networks.

Unlike traditional and many recently introduced protocols, ZMODEM

safeguards all data and supervisory information with effective error

detection.

ZMODEM rapidly transfers files, particularly with buffered (error

correcting) modems, timesharing systems, satellite relays, and wide area

packet switched networks.

User Friendliness is an important ZMODEM feature.  ZMODEM AutoDownload(TM)

(Automatic file Download initiated without user intervention) greatly

simplifies file transfers compared to traditional protocols.

DSZ's ZMODEM provides advanced file management features including

Intelligent Crash Recovery, flexible control of selective file transfers,

and security verified command downloading.

DSZ is one of many available programs that implement the ZMODEM file

transfer protocol.