****************************************************************

COMMUNICATION INTERRUPTS

September 5th, 1987

By Dale Loftis

****************************************************************

This archive should contain the following files:

COMINT.C                Test program and queue routines.

COMINT.PRJ              Turbo C project link list.

INT3.ASM                COM 2 interrupt assembly language routines.

INT4.ASM                COM 1 interrupt assembly language routines.

****************************************************************

After finding a need for RS232 interrupt routines and not being able to locate

anything suitable on any BB's, I decided to write these routines and put them

on the service for others.

The INT3 and INT4 routines are the real heart of the routines as they place

the incoming and outgoing characters in the proper places.

I have succesfully run programs using these routines up to 9600 baud on a AT

and had no problems with losing characters.

If you decide to alter these routines be aware that using INTerrupts to place

the characters in queues like these routines do,  it is imperative that you

act upon the characters by bumping the appropriate pointers depending on

whether your transmitting or receiving.

****************************************************************

com1_set_interrupt(parameters);  initializes port and sets interrupt vector

com2_set_interrupt(parameters);  initializes port and sets interrupt vector

com1_restore_interrupt();   restores interrupt vector in disables interrupts

com2_restore_interrupt();   restores interrupt vector in disables interrupts

****************************************************************

The com_status() routine displays the status of the program as it runs on the

line 24 of the screen and is formatted as follows:

aaaa - bbbb   cccc - dddd  ee  ff  gggg - hhhh  iiii - jjjj  kk  ll

where -

RECEIVE

aaaa =  *   COM 1 in  queue pointer   (count of characters received)

bbbb =  *   COM 1 out queue pointer   (count of received chars acted upon)

TRANSMIT

cccc =  *   COM 1 in  queue pointer   (count of chars to be transmitted)

dddd =  *   COM 1 out queue pointer   (count of chars that were transmitted)

INT STATUS

ee  =      COM 1 last interrupt status

00 = modem status interrupt

01 = xmit holding register empty interrupt

02 = data received interrupt

03 = reception error interrupt

PORT STATUS

ff  =      COM 1 modem status register status

bit 7   1 = "data carrier detect"

bit 6   1 = "ring indicator"

bit 5   1 = "data set ready"

bit 4   1 = "clear to send"

bit 3   1 = change in "data carrier detect"

bit 2   1 = change in "ring indicator"

bit 1   1 = change in "data set ready"

bit 0   1 = change in "clear to send"

****************************************************************

RECEIVE

gggg =  *   COM 2 in  queue pointer   (count of characters received)

hhhh =  *   COM 2 out queue pointer   (count of received chars acted upon)

TRANSMIT

iiii =  *   COM 2 in  queue pointer   (count of chars to be transmitted)

jjjj =  *   COM 2 out queue pointer   (count of chars that were transmitted)

INT STATUS

kk  =      COM 2 last interrupt status

00 = modem status interrupt

01 = xmit holding register empty interrupt

02 = data received interrupt

03 = reception error interrupt

PORT STATUS

ll  =      COM 2 modem status register status

bit 7   1 = "data carrier detect"

bit 6   1 = "ring indicator"

bit 5   1 = "data set ready"

bit 4   1 = "clear to send"

bit 3   1 = change in "data carrier detect"

bit 2   1 = change in "ring indicator"

bit 1   1 = change in "data set ready"

bit 0   1 = change in "clear to send"

****************************************************************

Items marked with a "*" are the actual offsets into the xmit and

receive queues and wrap back to zero when the queue sizes are reached.

****************************************************************

The variables com1_rs232_error and com2_rs232_error contain the status from a

reception error interrupt and is the line status register, formatted as:

bit 7   1 = time-out (off-line)

bit 6   1 = xmit shift register empty

bit 5   1 = xmit holding register empty

bit 4   1 = break detect

bit 3   1 = framing error (transmission is out of sync)

bit 2   1 = parity error

bit 1   1 = received data overrun (char not read in time)

bit 0   1 = byte of data has been received

****************************************************************