Removable drives under OS/2

=============================================

Removable drive here is:

- removable hard disk drive (device type is 0: DASD) or

- MO (device type is 7: OPTICAL).

A removable drive works as a floppy or a hard drive under OS/2 2.x, Warp.

As Floppy drive

===============

- removable medium is treated as a big floppy

- can't use FDISKPM & FDISK commands, but FORMAT is O.K.

- data can be changed between OS/2 & Win95 or DOS => media must be formatted

under OS/2 first

A removable drive was used as a big floppy drive under OS/2 2.x and Warp.

TMSCSIM.ADD and TMSCSIW.ADD reports it as SCSI type 0 (DASD) to OS2DASD

that treats it like a high capacity floppy. FDISKPM or FDISK will not see

the removable drive when works as Floppy, but FORMAT will work here.

1). DOS <-> OS/2

The removable media formatted under DOS may not be read/written under

OS/2. To make removable media data be exchangable between OS/2 and DOS,

the media must be formatted under OS/2.

2). Win95 <-> OS/2

The removable media formatted under windows 95 may not be read/written

under OS/2. To make removable media data be exchangable between OS/2 and

windows 95, the following lines must be included in OS/2 CONFIG.SYS:

BASEDEV=TMSCSIM.ADD /MO /HDMO /A:0

BASEDEV=OS2DASD.DMD

BASEDEV=OPTICAL.DMD

where /MO is for device type 7 (OPTICAL),

/HDMO is for device type 0 (DASD),

OPTICAL.DMD can be downloaded from IBM web server at:

http://www.europe.ibm.com/getdoc/psmemea/progserv/device/storagea.html

As Hard drive

=============

- removable medium is treated as a fixed hard drive (multiple partitions

is allowed, but can NOT change media)

- can use FDISKPM, FDISK and FORMAT commands

- data can only be changed between OS/2 & DOS (Not Win95)

A removable drive can be used as a hard drive if LOCKDRV.FLT is installed

in the OS/2 CONFIG.SYS file:

BASEDEV=TMSCSIM.ADD

BASEDEV=LOCKDRV.FLT /UNIT:0

(Note: Once the LOCKDRV.FLT is loaded, no media change is allowed. To change

the media you must re-boot the system.)

When removable drive acts as a hard drive, FDISKPM or FDISK will see it and

can make partitions. The media partitioned and formatted under DOS can

be read/written under OS/2, vice-versa. However, in this case removable

media data will not be exchangable between OS/2 and windows 95.

In the above examples, TMSCSIM.ADD was used for controller DC-390(T).

For controller DC-390W/F/U, TMSCSIM.ADD should be replaced with TMSCSIW.ADD.