**ATABLET.DOC file 02/14/94

ATABLET Mouse Driver V1.09

==========================

ATABLET.COM driver provides the look and feel of absolute positioning

with most DOS applications which only support a relative positioning

input device, such as a mouse, mouse pen, track ball, etc. Once this

driver is installed, you can simply move between Windows and DOS

applications without having to change the tablet configuration.

This driver also includes the functions of Real Mode ADI driver for

using with the AutoDesk DOS applications which support the Real Mode ADI

driver. With the ATABLET.COM mouse driver, you don't need another

Real Mode ADI driver any longer.

This feature is helpful for eliminating some interference problems if

you used to change the working environments between the DOS application

using mouse driver and the AutoDesk application using another Real Mode

ADI driver.

Before using the ATABLET Mouse driver

-------------------------------------

Refer to the manual of your DOS application(s) to configure the input

device by choosing a Microsoft Mouse or Mouse Systems Mouse in Setup

of each application. Load the ATABLET.COM (see below) before you

start your DOS application. For most DOS applications, you will now be

able to use your ACECAD tablet as a mouse with the look and feel of

absolute positioning. (NOTE: Some DOS applications do not support

absolute positioning and, thereby, force relative positioning.) The

ATABLET.COM driver is compatible with Microsoft Mouse driver and will

automatically detect the size of the tablet on the assigned COM port.

Once you load the ATABLET driver, it remains in memory until you

reboot your computer or turn it off. If you load another mouse

driver after loading ATABLET.COM, ATABLET.COM will be invalid.

Before using the Real Mode ADI functions

----------------------------------------

Please refer to the manual of your AutoDesk application(s) to configure

the digitizer by choosing the Real Mode ADI in the configuration menu or

by modifying the appropriate environment setting file. Before you start

the AutoDesk application(s), load the ATABLET.COM with optional parameter

(see below) to activate the Real Mode ADI functions.

If you load any other Real Mode ADI driver after loading ATABLET.COM,

the Real Mode ADI functions of ATABLET.COM will be invalid but leave

the mouse driver functions available for using with DOS applications.

Loading the ATABLET Mouse driver

--------------------------------

First, you must install the ATABLET.COM onto your hard disk by executing

the INSTALL.EXE in the root directory of this diskette. Before you do,

please read the README.DOC file in the root directory of this diskette

for the details about the installation.

To load the ATABLET driver, change to the ACECAD directory (created

by INSTALL.EXE) by typing the following command (where C: = the

designation for your hard disk) at the DOS prompt and press <Enter>.

CD C:\ACECAD

Then type the following command and press <Enter>.

ATABLET [/n][/n][...

where /n is an optional parameter(s) described below. This same

command can be placed in your AUTOEXEC.BAT file to load ATABLET at

start up.

The directory path for ATABLET ("C:\ACECAD" in above command example)

may be added to your DOS PATH statement in your AUTOEXEC.BAT file.

See your DOS documentation for more information on the use of the

PATH statement.

NOTE :  Both modifications of the AUTOEXEC.BAT file mentioned above may

has been made by the INSTALL.EXE program if you asked.

ATABLET displays sign-on and status messages indicating the current

parameter settings for Mouse mode, Transducer type, Button Assignments,

COM port, Real Mode ADI status and Coordinate Mode each time it is

executed.

PARAMETER DESCRIPTIONS

----------------------

1)  Serial Port assignment   ---  /#

Assigns the port where the tablet is installed.

/# selects the COM port that the tablet is connected to.

/1  --- Selects COM1

/2  --- Selects COM2

/3  --- Selects COM3

/4  --- Selects COM4

If a COM port is not specified, the driver will attempt to

automatically detect which COM port your ACECAD Tablet is

connected to. We recommend setting the port assignment explicitly

by using this parameter.

2)  IRQ Number Setting  --- /I#

This parameter allows you to specify the IRQ number.  The IRQ

number is generally standard for each COM port (COM1 to COM4) for

most serial I/O cards, so the user normally does not need to

select the IRQ number. If necessary, please refer to the manual of

your serial I/O card to select the exact IRQ number with the

serial port assignment. The IRQ number (#) is a decimal number in

the range of 2 to 5.

3)  Coordinate Mode Setting  ---  /A or /R

This specifies whether the driver communicates in Absolute or

Relative mode.

/A  --- Specifies Absolute mode

/R  --- Specifies Relative mode (default)

If your DOS application does not support Absolute mode, the

cursor movement will still be in Relative mode even though the

coordinate mode in the driver is set to Absolute mode.

4)  Mouse Mode --- /MS or /PC

This specifies whether the driver is set to be compatible with

Microsoft Mouse (2-button Mouse) or Mouse Systems Mouse (3-button

Mouse).

/MS --- Sets the driver to Microsoft Mouse. (default)

/PC --- Sets the driver to Mouse Systems Mouse.

If necessary, select the same Mouse device in the setup of your

DOS application by referring to the manual of the application.

If your DOS application supports only a Microsoft Mouse device,

you must set the driver to Microsoft Mouse (default). The Middle

button in button assignments only applies to Mouse Systems Mouse

mode.

5)  Button Assignments --- /B

The /B parameter allows you to select the transducer type you use

and the set of button assignments you want. When you load the

driver with the /B option, the following table will appear on the

screen.

1. 2-button Stylus-Pen (default)

2. 3-button Stylus-Pen

3. 4-button Cursor-Puck

4. 16-button Cursor-Puck

(Press <Esc> to cancel)

Enter your transducer type (1):

Enter the number (1 to 4) to select the transducer type you use

and press <Enter>. Then follow the directions on the screen to

enter a letter to select the new set of button assignments you

want and press <Enter>. The selected transducer type and new set

of button assignments will be saved to the driver for use, so you

do not need to load the driver with the /B option every time. The

factory default set of button assignments in the driver is set A

for 2-button Stylus-Pen (Tip button is left click; Barrel button

is right click).

6)  Tracking Area   ---  /C

The /C parameter allows you to change the tracking (active) area

within the whole active area of your tablet. The default tracking

area is the whole active area of your tablet (5"x5" for ACECAT,

12"x12" for D-9000 and D-9000+, or 12"x18" for D-8000). If you use

the driver with your application software under relative mode, we

recommend that you use the whole active area as tha tracking area.

If you can use the driver with your application software under

absolute mode, the position of the stylus-pen or cursor-puck on

the tracking area corresponds directly to the screen position. If

you are using the tablet primarily for menu picking and cursor

movement, you may wish to reduce the size of the tracking area.

With a smaller tracking area, you can move from one corner of the

screen to the other with a shorter distance on the tablet,

substantially increasing your speed and ease of work.

To specify a new tracking area:

1.  When you load the driver with /C option, the configuration

menu will appear on the screen, which shows the active area

rectangle, message area and five function icons.

2.  Use the left button to choose the CONFIG icon. A dash-lined

rectangle will appear in the lower-left corner of the whole

active area rectangle, and the position of the pointing device

(the X and Y coordinates) on the tablet will appear on the

lower-right corner of the screen.

3.  Use the left button to click the lower-left corner of the new

tracking area, which you want to use, within the dash-lined

rectangle. Then click the upper-right corner of the new

tracking area.

4.  Choose any one of the following buttons;

OK button     -- Save the new tracking area to the driver and

then load the driver.

CANCEL button -- Cancel the new tracking area you chose.

Load the driver with the preset tracking area.

RESET button  -- Restore the tracking area to the whole active

area of the tablet.

TEST button   -- Test and try to use the new tracking area on

the screen of the configuration menu.

7)  LCD Screen Update Skip Setting   ---  /N#

This parameter is useful for some slower display systems. For

example, LAPTOP Systems or NOTEBOOK Systems which incorporate

an LCD display.

The # is a decimal number in the range of 0 - 100 that specifies

the threshold value for cursor drawing. The cursor is drawn on

screen once the accumulated count value of transducer movement

reaches the threshold value. Larger values will reduce the

sensitivity of the driver. Default is /N0.

8)  Text Cursor Lock Function  ---  /O#

This parameter allows the user to inhibit application programs

from modifying the cursor attribute on the screen when in text

mode. When enabled, the cursor is locked to the default text

cursor.

/O1 ---- Enable the cursor locked function

/O0 ---- Disable the cursor locked function (default)

9)  Sensitivity Setting --- /S#, /H# or /V#

This allows the user to change the tracking response in relative

mode. You may want to experiment to find a comfortable setting.

/S  --- Specifies both Horizontal & Vertical Sensitivity in one

parameter

/H  --- Specifies Horizontal Sensitivity

/V  --- Specifies Vertical Sensitivity

The # is a decimal number in the range of 1 - 100. Default value

is 50. Larger values result in a faster response and lower values

are slower, great for highly accurate cursor pointing.

10) Disable/Remove Driver --- /OFF

This parameter removes the resident portion of the driver from

system memory if possible, and disables driver functionality

until it is reloaded again.

NOTE: The driver can not be physically removed from memory if

another "memory resident" program has been loaded after

ATABLET. However, ATABLET is still disabled (turned off)

with the /OFF parameter.

11) Activate Real Mode ADI functions --- /ADION

This parameter activate the Real Mode ADI functions for using

this driver with AutoDesk application(s) which support Real Mode

ADI driver.

NOTE : The Real Mode ADI functions use the system interrupt numbered

79H of your cmputer (where H means the number is represented

in hexadecimal notation).

12) Deactivate Real Mode ADI functions --- /ADIOFF

This parameter deactivate the Real Mode ADI functions but the

mouse driver functions are still available for DOS applications.

CHANGING PARAMETERS ONCE ATABLET IS LOADED

------------------------------------------

Parameters may be changed after loading ATABLET by issuing another

ATABLET command. For example, you may want to experiment with several

different /S or /N values to determine the value you like best. The

parameters can be combined in any order, except that Sensitivity

parameters only work in Relative Mode and /OFF should be issued alone.

COMMAND EXAMPLES

----------------

C:\>ATABLET /1/A

This command loads the ATABLET driver and sets it to communicate

with COM1 in Absolute mode.

C:\>ATABLET /2/N10

ATABLET is loaded for COM2 in Relative mode (default), Sensitivity

of 50 (default), and a skip value of 10.

C:\>ATABLET /OFF

This disables ATABLET and removes it from resident memory, if

possible.

C:\>ATABLET /3/I5

ATABLET is loaded for COM3 with IRQ number 5.

C:>ATABLET /2/ADION

ATABLET is loaded for COM2 and the Real Mode ADI functions are

activated.

C:>ATABLET /ADIOFF

The Real Mode ADI functions are deactivated but the mouse driver

functions are still available for DOS applications.

TROUBLESHOOTING

---------------

Problem:  You can not load the driver and an error message appears.

Solution: Please check your cable connections. If the tablet is

connected to COM3 or COM4, please also check whether the COM

port and its IRQ number on your serial card are the same as

the parameters used for the ATABLET driver.

Problem:  The cursor responds to the movement of your pointing device

sometimes, but not always.

Solution: Check to see if the tracking area is set for the full active

area. The tracking area is defined through the driver with

the /C parameter.

Problem:  Button functions are not correct in your application

software.

Solution: Please check that the set �#-�#/##1 #3@#5`#7�#9�#;�#=�#?##h

the driver is what you want. Also, make sure that y�#m�#o##q #s@#u`#w�#y�#{�#}�###use device in setup of your appl	��	��	��	��	�#

�

�@

�`

��####mode (Microsoft Mouse or Mouse Systems Mouse) of the

driver.